## Features

- Compatible with EOBD/OBDII standard

- RS232 Communication till 115200 Baud

- EOBD Modes 1..9 / J2012 modes 10-FF

- Hardware and software handshaking

- ISO9141-2 , KWP2000

- J1850 VPWM and PWM

- CAN ISO15765 and J1939-71 messaging

- ALDL8192 Support

- KW1281 , KW71 , KW82 support

- Multiple response from multiple ECU

- Physical and fonctional addressing

- 3 separate receiving CAN Buffer

## Description

OBDII is an international standard for communication between automobiles and diagnostic testers.it specifies a serial data communication bus between ECUs and diagnostic test SAE OBDII Scan Tool ( SAE j1978 ).

The OE90C2700 provides an inteligent interface between a Host and the vehicle’s Electronic control units. Both LEDs display the current communication status and connected information.

OE90C2700 is developed to meet the standard with ISO9141,KWP2000,SAE j1850 , ALDL , j1939 KW1281 ,KW71 , KW82 and ISO15765 CAN protocols .

Multiprotocol to RS232 gateway

# OE90C2700

When fitted on a 2600 PCB this chip will work but needs some modification of hardware for full fonctions.

Pin description

| Pin | Pin Name | Type | Description |
| --- | --- | --- | --- |
| 1 | Vref | I | Connect to 2.5 V ref when measuring Vbat |
| 2 | GND |  | Ground |
| 3 | VCC |  | Supply voltage |
| 4 | RXCAN | I | CAN BUS input |
| 5 | TXCAN | O | CAN BUS output |
| 6 | LED1 | O | OK LED max 5 mA for low current LED |
| 7 | TXPWM- | O | Transmit (-) output for PWM signal |
| 8 | TXPWM+ | O | Transmit (+) output for PWM signal |
| 9 | CLKIN | I | İnput for external Baudrate generator for RS232 |
| 10 | ALDLin | I | ALDL Input |
| 11 | ALDLout | O | ALDL output |
| 12 | RXPWM | I | PWM input |
| 13 | TXD | O | RS232 output |
| 14 | RXD | I | RS232 input |
| 15 | XTAL2 | I | 16 Mhz crystal input |
| 16 | XTAL1 | I | 16 MHz crystal input |
| 17 | VCC |  | Supply voltage |
| 18 | GND | I | Ground |
| 19 | RESET | I/O | A high level on this pin during 2 machine cycles while the oscillator is running resets the device. |
| 20 | LED2 | O | LED output to indicate the frames exchange |
| 21 | RxL-Vadc | I | ISO L-Line input / analog channel input When used as dual K line dont use the ADC fonction |
| 22 | RXK | I | ISO K-line input |
| 23 | TXL | O | ISO L-Line output |
| 24 | TXVPWM | O | VPWM transmit output |
| 25 | RXVPWM | I | VPWM receive input |
| 26 | TxK | O | ISO K-Line output |
| 27 | DOWNLD | I | A low on this pin while power on puts the device in download mode |
| 28 | SYSCLK | I | System clock for RS232 baudrate |

# Introduction to 2700

The 2700 is based on a ATMEL T89C51CC02 microcontroller running in X2 mode that allows a 32 Mhz internal frequency at 16 MHz standard crystal .

This chip is provided to convert all known protocol to RS232 format. Because of its pin compatibility with OE90C2600 industry standard IC , it will work on a mOByDic 2600 PCB. Only the Battery voltage measuring , Dual K line and ALDL fonctions are not directly realisable on this PCB.

The main difference against 2600 is the protocol searching at power on .Once powered via  OBD connector , blink the yellow and green LED alternately to indicate the readiness of the chip. The user needs to start ‘ seach for protocol’ fonction for a succesfully connection to car.

Some access to different ECUs are possible via an external multiplexer of OBD Pins. For exemple accessing an OPEL ECU with KW82 protocol needs to move the K-Line from pin 7 to pin 12 on OBD connector or via multiplexer

While using a 2600 board , an external 7815 regulator will be needed for J1939/71 operation

A Dual K line concept is implemented. On a 2600 pcb works only 1 dual K-line ( pin 7 ). For ALDL fonction the user should add some external component to ALDL in/output and connect the output/input to OBD pin 9 in order to get this protocol fonctional.

While measuring the Battery voltage , the user has to connect a 2.5 V ref voltage ( available on pin 5 of 82C251 CAN transceiver ) and he has to build a voltage divider with 2 resistors.

# Software migration from 2600 to 2700

Software developer need to check the chip ident and if 2700 then they have to send a send seach for protocol command ( 13 ) .

All command with color yellow are compatible with 2600. All command with green color are new but compatible to 2600. All red command can only work on 2700 PCB. Note that it is possible to get all new commands are realisable with a little Hardware modification on the 2600 Board.

A new seed/key algorithm is implemented in 2700 to allow the software developer  to protect his software against crack. An EEPROM Area is provided to user for own serial number registration.

All default values of 2700 are set to work in 2600 mode. The software developer has to care the protocol search phase at the beginning. The 2700 does not start to seach for protocol automatically.

The yellow LED indicates a TX/RX message eighter on OBD BUS or on RS232.

If the software developer wait 75 ms after the first byte ( when ISO/KWP auto keep alive fonction is active )  while sending a RS232 string to 2700 no hardware and no software Handshaking will be necesarry.

The 2700 chip is not VAG COM compatible but it can communicate with VW group cars via KW1281 and VAG-CAN using appropriate commands.

Note that all the protocols in 2700 are adjustable to your car in order to send the corresponding messages to the car. An autobaud mode is implemented to K-line and CAN BUS to detect automatically the correct baudrate.

Handling Multiple ECUs

The software developer has to give to user the possibility to select working ECU. He has to set the corresponding header and Filter automatically. The big problem when a software developer uses fonctional messaging , he gets the multiple message as response . It is not always in the same order of message . The responding ECU address in the respons string should be tested. After getting all the presents ECUs , the software developer has to set the corresponding message filter and header for using a physical addressing instead of fonctional addressing to avoid the data garbage.

Hardware application notes

## Typical connection of oe90c2700

Typical Power supply

Typical PWM interface circuit

Typical VPWM interface circuit

Typical CAN interface circuit

Typical Dual K-Line interface circuit

Typical Vbat measuring interface circuit

With this circuit it is possible to measure the Vbat voltage. Note that Vadc is max 2.5 V.

Measuring range with these resistor is 0..16 Volt. Please connect Vref input ( pin 1 ) of 2700 to pin 5 of the PCA82c251 in order to feed the Vref input with 2.5V reference voltage

## Typical ALDL interface circuit

Aldl interface can be realised with the second no used part of MAX232 driver. Note that a non standard pin of OBD connector is used for this purpose.

## SOFTWARE INTERFACING TO CHIP

OE90C2700 communicates via RS232 with 9600/8/n/1 default setting parameter PC is master and mOByDic is slave. Message are based on query/reply structure . The chips gives never starts communication itself. The communication works on half duplex basis. The 2700 has a communication timeout of 2 sec. after the 1. byte of message is received..  A RS232 buffer of 256 bytes is provided.

RS232 device driver routines

İf the user sends a 1 to mOByDic , mOByDic responds with 1 . This function allows a line checking or device connected checking. Some delphi exemples show how to begin to communicate with mOByDic. Please note that the following routines dont intend to be used in a professional application , they only allow the user to understand the messaging strategy of mOByDic 2700.

unit mOByDic_device_driver;

interface

uses

Windows, Messages, SysUtils, Classes, Graphics, Controls, Forms,

Dialogs, StdCtrls;

var

hCommFile     : THandle;

DCB           : TDCB;			    // structure for rs232

read_Error    : boolean;			    // RS232 Read error by timeout

RS232_Buffer  : array [0..255] of byte;  // buffer for incoming bytes

implementation

//******************************************************************************

//*                                                                            *

//* Function: open and initialize COM1 port  9600 baud / 8 / n / 1             *

//* Input   : ---------                                                        *

//* Output  :                                                                  *

//*                                                                            *

//******************************************************************************

Procedure Open_COMM;

begin

// change COM1 if another port will be used

hCommFile := CreateFile('COM1' , GENERIC_READ or GENERIC_WRITE,

0 , Nil, OPEN_EXISTING, FILE_ATTRIBUTE_NORMAL, 0);

GetCommState(hCommFile,DCB);    // set comm parameter

with DCB do

begin

Baudrate:= CBR_9600;

ByteSize:= 8;

Parity  := NOPARITY;

end;

SetCommState(hCommFile,DCB);

if hCommFile = INVALID_HANDLE_VALUE then  // error at open port

begin

ShowMessage ('ERROR OPENING PORT');

exit;

end;

end;

//******************************************************************************

//*                                                                            *

//* Function: close a open comm port                                           *

//* Input   :                                                                  *

//* Output  :                                                                  *

//*                                                                            *

//******************************************************************************

Procedure close_COMM;

begin

closeHandle(hCommFile);

end;

//******************************************************************************

//*                                                                            *

//* Function: clearBuffers, clear rs232 Tx and Rx buffers                      *

//* Input   :                                                                  *

//* Output  :                                                                  *

//*                                                                            *

//******************************************************************************

Procedure clearBuffers;

begin

purgecomm(hcommfile,PURGE_TXCLEAR);

purgecomm(hcommfile,PURGE_RXCLEAR);

end;

//******************************************************************************

//*                                                                            *

//* Function: write a byte to open RS232 port no timeout occurs                *

//* Input   : tosend : byte to send                                            *

//* Output  :                                                                  *

//*                                                                            *

//******************************************************************************

Procedure write_RS232 ( tosend:byte);

Var sent:dword;

begin

WriteFile (hCommFile,tosend,1,sent,nil);

end;

//******************************************************************************

//*                                                                            *

//* Function: Read a byte from RS232                                           *

//* Input   : Timeout value in mS to wait on first byte                        *

//* Output  : received char  , Read_error flag is true if any error            *

//*                                                                            *

//******************************************************************************

Function read_RS232(Timeout_value:word) : byte;

var

TimeOutBuffer:PCOMMTIMEOUTS;

bytesread:dword;

read_byte:byte;

begin

//timeout parameter setting

GetMem(TimeoutBuffer,sizeof(COMMTIMEOUTS));

GetCommTimeouts (hCommFile,TimeoutBuffer^);

TimeoutBuffer.ReadTotalTimeoutMultiplier:=0;

TimeoutBuffer.ReadTotalTimeoutConstant:=timeout_Value;

SetCommTimeouts (hCommFile,TimeoutBuffer^);

ReadFile(hCommFile,Read_byte,1,bytesread,nil);

if (BytesRead = 0)

then begin

Read_RS232:= 0;

read_error := true;

end

else begin

read_error := false;

Read_RS232:= read_byte;

end;

end;

end.

# General purpose commands

| Commande | 01 |
| --- | --- |
| Fonction | First contact to 2700 / line checking |
| Remarque | This command allows to begin a communication with 2700. It is used mostly to test if mobydic unit is connected or not. The unit must be plugged into OBD connector in order to get power from this connector. |
| Request | 01 |
| Pos. Response | 01 |
| Neg. Response | N/A |

Exemple for command 01

Open_COMM;

clearbuffers;

write_rs232(1); 			    // send first contact message

rs232_buffer[0]:=read_rs232(100);    // get the response with 100 ms timeout

if read_error

then ........                   // no contact

else if rs232_buffer[0]=1

then ........            // first contact OK device existes

else ........            //  no contact

| Commande | 03 |
| --- | --- |
| Fonction | Read serial number of 2700 |
| Remarque | For own serial number please use command 7 and 8 |
| Request | 03 |
| Pos. Response | 03 HB LB HB = high byte of serial number LB = low byte of serial number Serial# = (256*HB) + LB Actually not used and give 00 00 back |
| Neg. Response | N/A |

Exemple for command 03

Open_COMM;

clearbuffers;

write_rs232(3); 			    // send read serial number command

rs232_buffer[0]:=read_rs232(100);    // get the response with 100 ms timeout

if read_error then exit              // no response

rs232_buffer[1]:=read_rs232(10);     // get the response with 10 ms timeout

if read_error then exit              // no response

rs232_buffer[2]:=read_rs232(10);     // get the response with 10 ms timeout

if read_error then exit              // no response

Serial:=(256*rs232_buffer[1])+rs232_buffer[2]; // get serial

| Commande | 04 |
| --- | --- |
| Fonction | Read version of 2700 |
| Request | 04 |
| Pos. Response | 04 HB LB HB = high byte version LB = low byte of version Version# = (256*HB) + LB ( xxx decimal => ver x.xx ) |
| Neg. Response | N/A |

Exemple for command 04

Open_COMM;

clearbuffers;

write_rs232(4); 			    // send read version number command

rs232_buffer[0]:=read_rs232(100);    // get the response with 100 ms timeout

if read_error then exit              // no response

rs232_buffer[1]:=read_rs232(10);     // get the response with 10 ms timeout

if read_error then exit              // no response

rs232_buffer[2]:=read_rs232(10);     // get the response with 10 ms timeout

if read_error then exit              // no response

Version:=(256*rs232_buffer[1])+rs232_buffer[2]; // get version in x100 format

| Commande | 05 |
| --- | --- |
| Fonction | Read found protocol |
| Request | 05 |
| Remarque | Red protocol number are destined to EOBD |
| Pos. Response | 05 HB LB HB = high byte of protocol LB = low byte of protocol Protocol = (256*HB) + LB 0x0000 : no protocol 0x0001 : ISO9141-2 keywords 08 08 0x0002 : ISO9141-2 keywords 94 94 0x0004 : KWP2000 slow init 0x0008 : KWP2000 fast init 0x0010 : J1850 PWM 0x0020 : J1850 VPWM 0x0040 : CAN 11 ident 250 KB 0x0080 : CAN 11 ident 500 KB 0x0100 : CAN 29 ident 250 KB 0x0200 : CAN 29 ident 500 KB 0x0400 : SAE J1939 0x0800 : KW1281 0x0801 : KW71 0x1000 : KW81 0x1001 : KW82 0x2000 : ALDL 0xFFFF : Protocol found by user initialization |
| Neg. Response | N/A |

Exemple for command 05

Open_COMM;

clearbuffers;

write_rs232(4); 			    // send read Protocol number command

rs232_buffer[0]:=read_rs232(100);    // get the response with 100 ms timeout

if read_error then exit              // no response

rs232_buffer[1]:=read_rs232(10);     // get the response with 10 ms timeout

if read_error then exit              // no response

rs232_buffer[2]:=read_rs232(10);     // get the response with 10 ms timeout

if read_error then exit              // no response

Protocol:=(256*rs232_buffer[1])+rs232_buffer[2]; // get protocol

| Commande | 06 |
| --- | --- |
| Fonction | Read chip ident of mOByDic |
| Request | 06 |
| Pos. Response | - HB LB HB = high byte of chip ident LB = low byte of chip ident Version# = (256*HB) + LB 2700 = OE90C2700 |
| Neg. Response | N/A |

Exemple for command 06

Open_COMM;

clearbuffers;

write_rs232(6); 			    // send read chip ident command

rs232_buffer[0]:=read_rs232(100);    // get the response with 100 ms timeout

if read_error then exit              // no response

rs232_buffer[1]:=read_rs232(10);     // get the response with 10 ms timeout

if read_error then exit              // no response

rs232_buffer[2]:=read_rs232(10);     // get the response with 10 ms timeout

if read_error then exit              // no response

chip_ident:=(256*rs232_buffer[1])+rs232_buffer[2]; // get chip ident

| Commande | 07 |
| --- | --- |
| Fonction | Read from internal EEPROM location of mobydic |
| Remarque | Read 1 byte from EEPROM location 0..1023 |
| Request | 07 , LocationHB , LocationLB |
| Pos. Response | 07 , byte@location EEPROM locations 0..1023 are valid |
| Neg. Response | NACK  Location > 1023 |

Exemple for command 07 how to read a byte from EEPROM location 0200

Open_COMM;

clearbuffers;

write_rs232(7); 			    // send read EEPROM cells command

write_rs232(hi(200)); 		    // from location 200

write_rs232(lo(200)); 		    //

rs232_buffer[0]:=read_rs232(100);    // get the response with 100 ms timeout

if read_error then exit              // no response

if rs232_buffer[0] <> 07 then exit   // surely a NACK

rs232_buffer[1]:=read_rs232(10);     // get EEPROM byte

if read_error then exit              // no response

| Commande | 08 |
| --- | --- |
| Fonction | Write to EEPROM location in mobydic |
| Request | 08 , LocationHB , LocationLB , byte to write |
| Pos. Response | 08 , Echo of written byte EEPROM locations 16..1023 are valid The locations 0..15 are reserved for system |
| Neg. Response | NACK  EEPROM location > 1023 or < 15 |

Exemple for command 08 write a 3 to the location 1000

Open_COMM;

clearbuffers;

write_rs232(8); 			    // send write EEPROM cells command

write_rs232(hi (1000)); 		    // to location 1000

write_rs232(lo (1000));

write_rs232(3); 			    // byte to write

rs232_buffer[0]:=read_rs232(100);    // get the response with 100 ms timeout

if read_error then exit              // no response

if rs232_buffer[0] <> 08 then exit   //

rs232_buffer[1]:=read_rs232(100);    // get the response with 100 ms timeout

if read_error then exit              // no response

if rs232_buffer[1] <> 03 then exit  //

...................                  // byte written succefully

| Commande | 09 |
| --- | --- |
| Fonction | Seed / key dongle fonction |
| Remarque | This command is provided for software developer to secure his host software. This command generates a 3 bytes random seed and wait for 3 bytes key from User. It is possible to query the seed every time By false entering of keys mobydics does a reset and blocks the communication for 60 sec. An algorithme for key calculation will be provided to software developer. |
| Request | 09 |
| Pos. Response | 09 , seed1 , seed2 , seed3 then send key1 , key2 , key3 receive ACK or NACK |
| Neg. Response | NACK Keys are false |

Exemple for command 09

Open_COMM;

clearbuffers;

write_rs232(9); 			    // get seeds command

rs232_buffer[0]:=read_rs232(100);    // get the response with 100 ms timeout

if read_error then exit              // no response

rs232_buffer[1]:=read_rs232(10);     // get the response seed1

rs232_buffer[2]:=read_rs232(10);     // get the response seed2

rs232_buffer[3]:=read_rs232(10);     // get the response seed3

------- compute keys ------------

write_rs232(key1); 			    // send key1

write_rs232(key2); 			    // send key2

write_rs232(key3); 			    // send key3

rs232_buffer[0]:=read_rs232(100);    // get the response with 100 ms timeout

if read_error then exit              // no response

if rs232_buffer[0] <> ACK  then exit;

| Commande | 10 |
| --- | --- |
| Fonction | Set Baudrate of RS232 |
| Remarque | Only in 2700 v2.xx 16000000 Baudrate = ----------------------------------------------- 64 * ( 65536 – ((256*divisorHB)+divisorLB)) Default setting is 9600 Baud |
| Request | 10 divisorHB , divisorLB |
| Pos. Response | 10 , ACK  response at new set baudrate |
| Neg. Response | N/A |

Exemple for command 10

Open_COMM;

clearbuffers;

write_rs232(10); 			    // set baudrate command

write_rs232(div_HB); 		    // set baudrate high byte

write_rs232(div_LB); 		    // set baudrate low byte

rs232_buffer[0]:=read_rs232(100);    // get the response with 100 ms timeout

if read_error then exit              // no response

if rs232_buffer[0] <> 10  then exit;

rs232_buffer[1]:=read_rs232(10);     // get the response with 100 ms timeout

if read_error then exit              // no response

| Commande | 11 |
| --- | --- |
| Fonction | Soft Reset of 2700 |
| Request | 11 |
| Pos. Response | ACK ( 06 ) |
| Neg. Response | N/A |

Exemple for command 11

Open_COMM;

clearbuffers;

write_rs232(11); 			    // send soft reset command

rs232_buffer[0]:=read_rs232(100);    // get the response with 100 ms timeout

if read_error then exit              // no response

if rs232_buffer[0]=ACK

then ...............           // reset done

| Commande | 12 |
| --- | --- |
| Fonction | Battery voltage monitoring |
| Remarque | Get the battery voltage as digital value 0..1023 This corresponds to 0..16V. Dont give a higher voltage that 2.5 V at the Vadc input. Selecting another resistor – divisor ratio gives another plage of measuring. |
| Request | 12 |
| Pos. Response | - , ADC_HB , ADC_LB |
| Neg. Response | N/A |

Exemple for command 12

Open_COMM;

clearbuffers;

write_rs232(12); 			    // set Battery monitoring command

rs232_buffer[0]:=read_rs232(100);    // get the response with 100 ms timeout

if read_error then exit              // no response

rs232_buffer[1]:=read_rs232(10);     // get the response with 10 ms timeout

rs232_buffer[2]:=read_rs232(10);     // get the response with 10 ms timeout

Vbat_adc := (256*rs232_buffer[1])+rs232_buffer[2];

Vbat     :=  16.00 * Vbat_adc / 1023  // voltage 0.00 to 16.00 volt

| Commande | 13 |
| --- | --- |
| Fonction | EOBD search for protocol |
| Remarque | This command allows the backwards compatibity to 2600. |
| Request | 13 |
| Pos. Response | 13 HB LB HB = high byte of protocol LB = low byte of protocol Protocol = (256*HB) + LB 0x0000 : no protocol 0x0001 : ISO9141-2 keywords 08 08 0x0002 : ISO9141-2 keywords 94 94 0x0004 : KWP2000 slow init 0x0008 : KWP2000 fast init 0x0010 : J1850 PWM 0x0020 : J1850 VPWM 0x0040 : CAN 11 ident 250 KB 0x0080 : CAN 11 ident 500 KB 0x0100 : CAN 29 ident 250 KB 0x0200 : CAN 29 ident 500 KB |
| Neg. Response | N/A |

Exemple for command 13

Open_COMM;

clearbuffers;

write_rs232(13); 			    // search for EOBD protocol

rs232_buffer[0]:=read_rs232(45000);  // get the response with 45 sec. timeout

if read_error then exit              // no response

rs232_buffer[1]:=read_rs232(10);     // get the response with 10 ms timeout

rs232_buffer[2]:=read_rs232(10);     // get the response with 10 ms timeout

protocol := (256*rs232_buffer[1])+rs232_buffer[2];

| Commande | 16 |
| --- | --- |
| Fonction | 5 Baud Init ISO9141 |
| Remarque | 5 Baud init as ISO9141 this fonction doesnt test the last inverted byte from ECU. So the user can init ISO BUS in 9141 or 9141-2 modes. A keep alive is starting automatically ( see fonction 18 ) |
| Request | 16 , ECU_address , Baudrate baudrate = 0 work with fix 10400 baud baudrate = 1 9600 Baud |
| Pos. Response | - , keyword1 , keyword2 |
| Neg. Response | NACK , error_code |

Exemple for command 16 how to init an ISO9141-2 BUS

Open_COMM;

clearbuffers;

write_rs232(16); 			    // 5 Baud Init command

write_rs232($33); 			    // EOBD address

write_rs232(00); 			    // only 10400 baud

rs232_buffer[0]:=read_rs232(6500);   // get the response with 6500 ms timeout

if read_error then exit              // no response

rs232_buffer[1]:=read_rs232(10);     // get the response with 10 ms timeout

rs232_buffer[2]:=read_rs232(10);     // get the response with 10 ms timeout

if rs232_buffer[0] = NACK

then ...........               // see error code why

| Error code | description |
| --- | --- |
| 00 | No 0x55 seen ( no response ) |
| 01 | Keywords frame error |
| 02 | İnit OK |

| Commande | 17 |
| --- | --- |
| Fonction | Set ISO parameters |
| Remarque | İnterbyte time 2..25 mS Default value for interbyte time is 10 mS |
| Request | 17 , Interbyte time |
| Pos. Response | - , ACK |
| Neg. Response | NACK  interbyte time > 2..25 |

Exemple for command 17 how to set interbyte time

Open_COMM;

clearbuffers;

write_rs232(17); 			    // Set ISO parameter command

write_rs232(5); 			    // 5 ms interbyte time

rs232_buffer[0]:=read_rs232(100);    // get the response with 100 ms timeout

if read_error then exit              // no response

if rs232_buffer[0] = NACK

then ...........               // error NACK

rs232_buffer[1]:=read_rs232(10);     // get the response with 10 ms timeout

| Commande | 18 |
| --- | --- |
| Fonction | Set ISO keep alive string |
| Remarque | This string is sent every 3 sec. Default string is 68 6A F1 01 00 Cksum is calculed by mobydic String length is max 10 bytes. Command active in v2.00 |
| Request | 18 , length , keep_alive string |
| Pos. Response | - , ACK |
| Neg. Response | NACK string > 10 bytes |

Exemple for command 18 how to set a keep alive string

Open_COMM;

clearbuffers;

write_rs232(18); 			    // Set ISO parameter command

write_rs232(5); 			    // 5 bytes follow

write_rs232($68); 			    //

write_rs232($6A); 			    //

write_rs232($F1); 			    //  EOBD keep alive string

write_rs232(01); 			    //

write_rs232(00); 			    //

rs232_buffer[0]:=read_rs232(100);    // get the response with 100 ms timeout

if read_error then exit              // no response

if rs232_buffer[0] = NACK

then ...........               // error string > 10 bytes

rs232_buffer[1]:=read_rs232(10);     // get ACK

| Commande | 19 |
| --- | --- |
| Fonction | Send a ISO string to ECU |
| Remarque | This string is sent directly to ECU , checksum is calculated by mobydic. Note that a max of 255 byte can be received. |
| Request | Send 19 to mobydic Wait for 06 (ACK ) after receiving ACK user has only 2 sec. to continue Send length , data see exemple |
| Pos. Response | - , length , ISO response_string if length =0 no response is received from ECU |
| Neg. Response | N/A |

Exemple for command 19 how to send a direct ISO message to ECU

write_rs232(19); 			    // Handshake

rs232_buffer[0]:=read_rs232(100);    //

if read_error then exit              //

if rs232_buffer[0] <> ACK then exit

write_rs232(5); 			    // 5 bytes follow

write_rs232($68); 			    //

write_rs232($6A); 			    //

write_rs232($F1); 			    //  EOBD get Mode 1 PID 0 data

write_rs232(01); 			    //

write_rs232(00); 			    //

rs232_buffer[0]:=read_rs232(250);    // wait for 1.byte

if read_error then exit              // no response

if rs232_buffer[0] <> 19 the exit    // false sync.

Length := read_rs232(10);            // get incoming message length

For i:=1 to length do                // if length <> 0

Begin

rs232_buffer[i]:=read_rs232(10); // read length x byte 10 ms timeout

End;

Buffer after receiving

| Rs232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 13 | Response to Command 19 ( 13 hex ) |
| [1] | 0A | Length of string |
| [2] | 48 | ISO response header 1 |
| [3] | 6B | ISO response header 2 |
| [4] | ECU_address | Responding ECU address |
| [5] | 41 | Response to mode 1 |
| [6] | 00 | Response to PID 0 |
| [7] | xx | Data 1 |
| [8] | xx | Data 2 |
| [9] | xx | Data 3 |
| [10] | xx | Data 4 |
| [11] | CKS | Checksumm |

| Commande | 20 |
| --- | --- |
| Fonction | Start keep Aliving on ISO9141 |
| Remarque | Default is auto keep aliving |
| Request | 20 |
| Pos. Response | - , ACK |
| Neg. Response | N/A |

Exemple for command 20 how to start keep aliving on ISO9141

Open_COMM;

clearbuffers;

write_rs232(20); 			    // Start auto keep alive

rs232_buffer[0]:=read_rs232(100);    // get the response with 100 ms timeout

if read_error then exit              // no response

rs232_buffer[1]:=read_rs232(10);     // get the response with 10 ms timeout

if rs232_buffer[1] = ACK

then ...........               // done

| Commande | 21 |
| --- | --- |
| Fonction | Stop keep Aliving on ISO9141 |
| Remarque | Default is auto keep aliving |
| Request | 21 |
| Pos. Response | - , ACK |
| Neg. Response | N/A |

Exemple for command 21 how to stop keep aliving on ISO9141

Open_COMM;

clearbuffers;

write_rs232(21); 			    // Stop auto keep alive

rs232_buffer[0]:=read_rs232(100);    // get the response with 100 ms timeout

if read_error then exit              // no response

rs232_buffer[1]:=read_rs232(10);     // get the response with 10 ms timeout

if rs232_buffer[1] = ACK

then ...........               // done

| Commande | 25 |
| --- | --- |
| Fonction | Set ECU Address on KWP2000 |
| Remarque | Default setting is $33 EOBD By setting 0x33 , mobydic handles it as a fonctional request. Any other ECU address are physical requests |
| Request | 25 , ECU_Address |
| Pos. Response | 25 , ACK ( 06 ) |
| Neg. Response | N/A |

Exemple  command 25 how setting the header in mObydic to 0x10

Open_COMM;

clearbuffers;

write_rs232(25); 			     // send set address command

write_rs232($10); 			     // send direct ECU address to mOByDic rs232_buffer[0]:=read_rs232(100);     // get the response with 100 ms timeout

if read_error then exit               // no response

rs232_buffer[1]:=read_rs232(100);     // get the response with 100 ms timeout

if read_error then exit               // no response

if rs232_buffer[1] = ACK

then ..................           // handle

| Commande | 26 |
| --- | --- |
| Fonction | Connect to ECU |
| Remarque | This command allows a immediat connection to ECU with KWP2000 fast init. This command uses default ECU address or address set by command 25. An Auto keepalive function is actived sending the tester present message to ECU. Setting the ECU Address to EOBD provides a keep aliving by sending mode 1 PID 0 request message . The StartComm String is automatically generated by the chip. Take care to not initialize an elready initialized ECU |
| Request | 26 |
| Pos. Response | 26 , KWP2000 startCOM positive / negative response |
| Neg. Response | NACK not connected please turn ignition key on |

Exemple  command 26 how connecting  to PCM via Diagnosis line

write_rs232(26); 			     // send connect command

rs232_buffer[0]:=read_rs232(100);     // get the response with 100 ms timeout

if read_error then exit               // no response

rs232_buffer[1]:=read_rs232(10);     // get the response with 10 ms timeout

if rs232_buffer[0] = NACK

then ...........               // no response from ECU

rs232_buffer[2]:=read_rs232(10);     // get the response with 10 ms timeout

| Commande | 27 |
| --- | --- |
| Fonction | Start keep Aliving on KWP2000 |
| Remarque | Default is auto keep aliving . if EOBD header then mobydic sends a service1 PID 0 message. Elsewhere mobydic send a 3E Tester present to ECU |
| Request | 27 |
| Pos. Response | 27, ACK |
| Neg. Response | N/A |

Exemple for command 27 how to start keep aliving on KWP2000

write_rs232(27); 			    // Start auto keep alive

rs232_buffer[0]:=read_rs232(100);    // get the response with 100 ms timeout

if read_error then exit              // no response

rs232_buffer[1]:=read_rs232(10);     // get the response with 10 ms timeout

if rs232_buffer[1] = ACK

then ...........               // done

| Commande | 28 |
| --- | --- |
| Fonction | Stop keep Aliving on KWP2000 |
| Remarque | Default is auto keep aliving |
| Request | 28 |
| Pos. Response | 28, ACK |
| Neg. Response | N/A |

Exemple for command 28 how to stop keep aliving on ISO9141

Open_COMM;

clearbuffers;

write_rs232(28); 			    // Stop auto keep alive

rs232_buffer[0]:=read_rs232(100);    // get the response with 100 ms timeout

if read_error then exit              // no response

rs232_buffer[1]:=read_rs232(10);     // get the response with 10 ms timeout

if rs232_buffer[1] = ACK

then ...........               // done

| Commande | 29 |
| --- | --- |
| Fonction | Direct Communicate with ECU – KWP2000 |
| Remarque | The header and checksum is set by mobydic the user has only to send the data field and length of this. If 0x33 is set as ECU address , a functional addressing is provided. |
| Request | Send 29 to mobydic Wait for 06 (ACK ) after receiving ACK user has only 2 sec. to continue Send length , data see exemple |
| Pos. Response | 29 , length , data length=0 indicating no data |
| Neg. Response | NACK ( length doesnt match ) |

This command is used to communicate with connected ECU in KWP2000 format.

A Normal KWP2000 Message Format

| Header Bytes |  |  | Data Bytes |  |  |  |  |  |  | checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length | Target | Source | 1 | 2 | 3 | 4 | 5 | 6 | 7 | CKS / CRC |
| 80+length | ECU address | F1 |  |  |  |  |  |  |  | CKS |

Requested format for mobydic2700

| Length | Data Bytes |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |

Note that Checksum is automatically calculated by modydic2700

Mobydic responds with

| Header Bytes |  |  | Data Bytes |  |  |  |  |  |  | checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length | Target | Source | 1 | 2 | 3 | 4 | 5 | 6 | 7 | CKS / CRC |
| 80+length | F1 | ECU addr. |  |  |  |  |  |  |  | CKS |

## Exemple  get RPM using RLI on FIAT

Note that user has to set first the ECU address to 0x10 by command 25 and send a command 26 to connect to ECU.

After this both commands :

| Length | Data Bytes |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 2 | 21 | 30 |  |  |  |  |  |

Mobydic responds after 29  , length with :

| Header Bytes |  |  | Data Bytes |  |  |  |  |  |  | checksum |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length | Target | Source | 1 | 2 | 3 | 4 | 5 | 6 | 7 | CKS / CRC |
| 85 | F1 | ECU Addr. | 62 | 30 | xx | xx |  |  |  | CKS |

write_rs232(29); 			     	// send communicate command / Handshake

rs232_buffer[0]:=read_rs232(100);     	// get the response with 100 ms timeout

if read_error then exit               	// no response

if rs232_buffer[0] = ACK

then begin

write_rs232(02); 		       // 2 bytes follow

write_rs232($21); 		       // get data by RLI

write_rs232($30); 		       // RLI for RPM

rs232_buffer[0]:=read_rs232(250); // response

if read_error then exit           // no response

rs232_buffer[1]:=read_rs232(10);  // response message length

for i:=2 to rs232_buffer[1]+1 do  // if length <> 0

begin

rs232_buffer[i]:=read_rs232(10); // get data

if read_error then exit          // no response

end;

end

else                                // surely a NACK check length!!!

After succesfully reception following data are in RS232_Buffer

| Rs232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 1D | Command 29 |
| [1] | 8 | String length |
| [2] | 84 | Kwp message header byte 1 |
| [3] | F1 | Kwp message header byte 2 |
| [4] | ECU Addr. | Kwp message header byte 3 |
| [5] | 61 | Response to 21 read by RLI |
| [6] | 30 | RLI for RPM |
| [7] | xx | RPM data |
| [8] | xx | RPM data |
| [9] | CKS | Checksumm of KWP2000 message |

| Commande | 30 |
| --- | --- |
| Fonction | Set Header and incoming message filter J1850 - PWM |
| Remarque | This message set the 3 byte header and 3 byte incoming filter on j1850 PWM BUS Default setting is 61 6A F1 for header and 41 6B 10 for incoming filter 10 is the ECU address , setting FF as 3. byte of incoming filter annulate the ECU address filtering (multiple response ) |
| Request | 30 , HDR1,HDR2,HDR3,IMF1,IMF2,IMF3 |
| Pos. Response | 30,ACK |
| Neg. Response | N/A |

Exemple for command 30 how To set an J1850-PWM Header / filter

write_rs232(30); 			    // send MESSAGE command

write_rs232($61); 			    // header 1

write_rs232($6a); 			    // header 2

write_rs232($F1); 			    // header 3

write_rs232($41); 			    // incoming message header 1 filter

write_rs232($6B); 			    // incoming message header 2 filter

write_rs232($18); 			    // incoming message header 3 ( only TCM )

rs232_buffer[0]:=read_rs232(250);    // wait for 1.byte

if read_error then exit              // no response

if rs232_buffer[0] = NACK then exit    //

rs232_buffer[1]:=read_rs232(10);     // wait for 2.byte

if read_error then exit              // no response

if rs232_buffer[1] <> ACK the exit    //

| Commande | 31 |
| --- | --- |
| Fonction | Set Header and incoming message filter J1850 – VPM |
| Remarque | This message set the 3 byte header and 3 byte incoming filter on j1850 VPW BUS Default setting is 68 6A F1 for header and 48 6B 10 for incoming filter 10 is the ECU address , setting FF as 3. byte of incoming filter annulate the ECU address filtering |
| Request | 31 , HDR1,HDR2,HDR3,IMF1,IMF2,IMF3 |
| Pos. Response | 31 , ACK |
| Neg. Response | N/A |

Exemple for command 31 how To set an J1850-VPW Header / filter

write_rs232(31); 			    // send MESSAGE command

write_rs232($68); 			    // header 1

write_rs232($6a); 			    // header 2

write_rs232($F1); 			    // header 3

write_rs232($48); 			    // incoming message header 1 filter

write_rs232($6B); 			    // incoming message header 2 filter

write_rs232($18); 			    // incoming message header 3 ( only TCM )

rs232_buffer[0]:=read_rs232(250);    // wait for 1.byte

if read_error then exit              // no response

f rs232_buffer[1] = NACK then exit    //

rs232_buffer[1]:=read_rs232(10);     // wait for 2.byte

if read_error then exit              // no response

if rs232_buffer[1] <> ACK then exit    //

| Commande | 32 |
| --- | --- |
| Fonction | Send a free message to J1850 PWM BUS |
| Remarque | This message is sent to the Modul pointed by command 30 User needs only to send the bytes in data field of j1850 message format. No header sending is necessary. MOByDic uses already existing header. |
| Request | 32 , data_length , <data> |
| Pos. Response | 32 , length , Header1 , 0xF1 , Ecu_address , <data> , CS |
| Neg. Response | Data_length byte doesnt match the data length |

Exemple for command 32 get the mode 1 PID 0  from  by command 30  pointed modul

write_rs232(32); 			    // send MESSAGE command

write_rs232(2); 			    // 2 Bytes follow

write_rs232($01); 			    // Mode 1

write_rs232($00); 			    //  PID 0

rs232_buffer[0]:=read_rs232(250);    // wait for 1.byte

if read_error then exit              // no response

if rs232_buffer[0] <> 32 the exit    // false sync.

Length := read_rs232(10);            // get incoming message length

For i:=1 to length do                // if length <> 0

Begin

rs232_buffer[i]:=read_rs232(10); // read length x byte 10 ms interbyte timeout

End;

In Rs232_buffer user has following data :

After succesfully reception following data are in RS232_Buffer

| Rs232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 20 | Command 32 |
| [1] | 0A | String length |
| [2] | 41 | J1850 header byte 1 |
| [3] | 6B | J1850 header byte 2 |
| [4] | ECU Addr. | J1850 header byte 3 |
| [5] | 41 | Response to mode 1 |
| [6] | 00 | PID 00 |
| [7] | xx | Data 1 |
| [8] | xx | Data 2 |
| [9] | xx | Data 3 |
| [10] | xx | Data 4 |
| [11] | CRC | CRC of J1850 message |

| Commande | 33 |
| --- | --- |
| Fonction | Send a free message to J1850 VPWM BUS |
| Remarque | This message is sent to the Modul pointed by command 31 User needs only to send the bytes in data field of j1850 message format. No header sending is necessary. MOByDic uses already existing header. |
| Request | 33 , data_length , <data> |
| Pos. Response | 33 , length , Header1 , 0xF1 , Ecu_address , <data> , CS |
| Neg. Response | Data_length byte doesnt match the data length |

Exemple for command 33 get the mode 1 PID 0  from  by command 31  pointed modul

write_rs232(33); 			    // send MESSAGE command

write_rs232(2); 			    // 2 Bytes follow

write_rs232($01); 			    // Mode 1

write_rs232($00); 			    //  PID 0

rs232_buffer[0]:=read_rs232(250);    // wait for 1.byte

if read_error then exit              // no response

if rs232_buffer[0] <> 33 the exit    // false sync.

Length := read_rs232(10);            // get incoming message length

For i:=1 to length do                // if length <> 0

Begin

rs232_buffer[i]:=read_rs232(10); // read length x byte 10 ms interbyte timeout

End;

In Rs232_buffer user has following data :

After succesfully reception following data are in RS232_Buffer

| Rs232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 21 | Command 33 |
| [1] | 0A | String length |
| [2] | 48 | J1850 header byte 1 |
| [3] | 6B | J1850 header byte 2 |
| [4] | ECU Addr. | J1850 header byte 3 |
| [5] | 41 | Response to mode 1 |
| [6] | 00 | PID 00 |
| [7] | xx | Data 1 |
| [8] | xx | Data 2 |
| [9] | xx | Data 3 |
| [10] | xx | Data 4 |
| [11] | CRC | CRC of J1850 message |

| Commande | 35 |
| --- | --- |
| Fonction | Set ECU Address on CAN BUS 11 bits ID |
| Remarque | Default setting is $7DF ( EOBD control modul ) |
| Request | 35 , ECU_Address_HB,ECU_address_LB |
| Pos. Response | 35 , ACK ( 06 ) |
| Neg. Response | N/A |

When using this command , an incoming filter is automatically set for response header.

if the user sets the header to 07E0 ( PCM ) the incoming filter is set to 7E8 by mobydic.

Any access to EOBD moduls with the header 7DF results the setting of the incoming filter to 7E8...7EF automatically. Adjusting incoming filter is done with command 36

Exemple for command 35 how setting the header in mObydic to TCM on CAN Bus

Open_COMM;

clearbuffers;

write_rs232(35); 			     // send set address command

write_rs232($07); 			     // send TCM address to mOByDic write_rs232($E1); 			     // send TCM address to mOByDic rs232_buffer[0]:=read_rs232(100);     // get the response with 100 ms timeout

if read_error then exit               // no response

rs232_buffer[1]:=read_rs232(10);     // get the response with 10 ms timeout

if read_error then exit               // no response

if rs232_buffer[1] = ACK

then ..................           // handle

| Commande | 36 |
| --- | --- |
| Fonction | Set Incoming Filter on CAN BUS 11 bit ID |
| Remarque | Default setting is $7XX ( EOBD control modul ) Please refer to datasheets of T89c51cc02 for mask setting |
| Request | 36 , ECU_Address_HB,ECU_address_LB |
| Pos. Response | 36 , ACK ( 06 ) |
| Neg. Response | N/A |

Exemple for command 36 how setting the mask in mObydic to TCM on CAN Bus

Open_COMM;

clearbuffers;

write_rs232(36); 			     // send set address command

write_rs232($07); 			     // send TCM address to mOByDic write_rs232($E9); 			     // send TCM address to mOByDic rs232_buffer[0]:=read_rs232(100);     // get the response with 100 ms timeout

if read_error then exit               // no response

rs232_buffer[1]:=read_rs232(10);      // get the response with 10 ms timeout

if read_error then exit               // no response

if rs232_buffer[1] = ACK

then ..................           // handle

| Commande | 37 |
| --- | --- |
| Fonction | Set Header on CAN BUS 29 bits ID |
| Remarque | Default setting is 18 DB 33 F1 ( EOBD control modul ) |
| Request | 37 , 4 byte header |
| Pos. Response | 37 , ACK ( 06 ) |
| Neg. Response | N/A |

Exemple for command 37 how setting the header in mObydic to TCM on CAN Bus

write_rs232(37); 			     // send set address command

write_rs232($18); 			     // send TCM address to mOByDic write_rs232($DA); 			     // send TCM address to mOBydic

write_rs232($18); 			     //

write_rs232($F1); 			     //

rs232_buffer[0]:=read_rs232(100);     // get the response with 100 ms timeout

if read_error then exit               // no response

rs232_buffer[1]:=read_rs232(10);     // get the response with 10 ms timeout

if read_error then exit               // no response

if rs232_buffer[1] = ACK

then ..................           // handle

| Commande | 38 |
| --- | --- |
| Fonction | Set Incoming Filter on CAN BUS 29 bit ID |
| Remarque | Default setting is 18 DA F1 FF ( EOBD control modul ) Please refer to datasheets of T89c51cc02 for mask setting |
| Request | 38 , Incoming Filter |
| Pos. Response | 38 , ACK ( 06 ) |
| Neg. Response | N/A |

Exemple for command 38 how setting the mask in mObydic to PCM on CAN Bus

write_rs232(38); 			     // send set address command

write_rs232($18); 			     // send PCM address to mOByDic write_rs232($DA); 			     //

write_rs232($F1); 			     //

write_rs232($FF); 			     // Accept all ECU address

rs232_buffer[0]:=read_rs232(100);     // get the response with 100 ms timeout

if read_error then exit               // no response

rs232_buffer[1]:=read_rs232(10);      // get the response with 100 ms timeout

if read_error then exit               // no response

if rs232_buffer[1] = ACK

then ..................           // handle

| Commande | 39 |
| --- | --- |
| Fonction | Set Baudrate to CAN BUS |
| Remarque |  |
| Request | 39 , Baudrate Baudrate is a byte 0 : auto detect by listening 1 : 250 kB 2 : 500 KB no default value existe |
| Pos. Response | 39 , ACK |
| Neg. Response | Baudrate > 2 |

Exemple for command 39 how setting the Baudrate in mObydic

Open_COMM;

clearbuffers;

write_rs232(39); 			     // send set baudrate command

write_rs232(0); 			     // Auto detect and set

rs232_buffer[0]:=read_rs232(100);     // get the response with 100 ms timeout

if read_error then exit               // no response

rs232_buffer[1]:=read_rs232(10);      // get the response with 10 ms timeout

rs232_buffer[2]:=read_rs232(10);      // get the response with 10 ms timeout if

Baudrate := (256*rs232_buffer[1])+rs232_buffer[2];

| Commande | 40 |
| --- | --- |
| Fonction | Read internal CAN controller Registers |
| Remarque | The User has to handle this command carefully. All the internal CAN registers of mobydic can be read out Please refer to datasheet of T89C51CC02 from atmel to reprogram this registers. |
| Request | 40 |
| Pos. Response | 40 , CAN Register array ( 37 bytes ) |
| Neg. Response | N/A |

| Array pos | Register name |
| --- | --- |
| 0 | CANEN |
| 1 | CANGIE |
| 2 | CANIE |
| 3 | CANIDM1 |
| 4 | CANIDM2 |
| 5 | CANIDM3 |
| 6 | CANIDM4 |
| 7 | CANSIT |
| 8 | CANIDT1 |
| 9 | CANIDT2 |
| 10 | CANIDT3 |
| 11 | CANIDT4 |
| 12 | CANPAGE |
| 13 | CANSTCH |
| 14 | CANCONCH |
| 15 | CANBT1 |
| 16 | CANBT2 |
| 17 | CANBT3 |
| 18 | CANGSTA |
| 19 | CANGCON |
| 20 | CANTIML |
| 21 | CANTIMH |
| 22 | CANSTMPL |
| 23 | CANSTMPH |
| 24 | CANTCON |
| 25..32 | CANMSG1..8 |
| 33 | CANTTCL |
| 34 | CANTTCH |
| 35 | CANGIT |
| 36 | CANTEC |
| 37 | CANREC |

# Exemple how to read the internal CAN register

write_rs232(40); 			     // get CAN Register command

rs232_buffer[0]:=read_rs232(100);     // get the response with 100 ms timeout

if read_error then exit               // no response

if rs232_buffer[0]=40

then begin

for i:=1 to 37 do

begin

CAN_Buffer[i]:=read_RS232[10];

end;

end

else ............               // error

| Commande | 41 |
| --- | --- |
| Fonction | Write to internal CAN controller Registers |
| Remarque | The User has to handle this command carefully. All the internal CAN registers of mobydic can be written Please refer to datasheet of T89C51CC02 from atmel to reprogram this registers. |
| Request | 41 , CAN Register array ( 37 bytes ) |
| Pos. Response | 41 , ACK |
| Neg. Response | N/A |

| Array pos | Register name |
| --- | --- |
| 0 | CANEN |
| 1 | CANGIE |
| 2 | CANIE |
| 3 | CANIDM1 |
| 4 | CANIDM2 |
| 5 | CANIDM3 |
| 6 | CANIDM4 |
| 7 | CANSIT |
| 8 | CANIDT1 |
| 9 | CANIDT2 |
| 10 | CANIDT3 |
| 11 | CANIDT4 |
| 12 | CANPAGE |
| 13 | CANSTCH |
| 14 | CANCONCH |
| 15 | CANBT1 |
| 16 | CANBT2 |
| 17 | CANBT3 |
| 18 | CANGSTA |
| 19 | CANGCON |
| 20 | CANTIML |
| 21 | CANTIMH |
| 22 | CANSTMPL |
| 23 | CANSTMPH |
| 24 | CANTCON |
| 25..32 | CANMSG1..8 |
| 33 | CANTTCL |
| 34 | CANTTCH |
| 35 | CANGIT |
| 36 | CANTEC |
| 37 | CANREC |

# Exemple how to write to the internal CAN registers ( data in  CAN_array )

write_rs232(41); 			     // get CAN Register command

for i:=1 to 37 do

begin

write_rs232(CAN_Buffer[i]);

end;

rs232_buffer[0]:=read_rs232(100);     // get the response with 100 ms timeout

if read_error then exit               // no response

if rs232_buffer[0]<>41 then exit      // error

rs232_buffer[1]:=read_rs232(10);      // get the response with 10 ms timeout

if read_error then exit               // no response

if rs232_buffer[1]<> ACK then exit    // error

| Commande | 42 |
| --- | --- |
| Fonction | CAN BUS Monitoring 11 bit ID |
| Remarque | This command listens the CAN BUS 11 ID without sending a request with set header and filter |
| Request | 42 , Timeout in mS ( high byte , low_byte ) Timeout max 800 mS if timeout > 800 mS  timeout is set to 800 mS |
| Pos. Response | 42 , length , data bytes 42 , 0  no data after timeout |
| Neg. Response | N/A |

# Exemple how to listen a CAN BUS 11 bit in rs232_buffer

write_rs232(42); 			     // listen CAN BUS data command

write_rs232(0); 			     //

write_rs232(250); 			     // timeout 250 mS

rs232_buffer[0]:=read_rs232(100);     // get the response with 100 ms timeout

if read_error then exit               // no response

rs232_buffer[1]:=read_rs232(10);     // get the response with 100 ms timeout

if read_error then exit               // no response

if rs232_buffer[1]<> 0

then begin

for i:=1 to length do

begin

RS232_buffer[i]:=read_RS232[10];

end;

end

else ............               // error

| Commande | 43 |
| --- | --- |
| Fonction | CAN BUS Monitoring 29 bit ID / J1939 Monitoring |
| Remarque | This command listens the CAN BUS 29 ID without sending a request message with set header and filter |
| Request | 43 , Timeout in mS Timeout max 800 mS if timeout > 800 mS  timeout = 800 mS |
| Pos. Response | 43 , length , data bytes 43 , 0  no data after timeout |
| Neg. Response | N |

# Exemple how to listen a CAN BUS 29 bit in rs232_buffer

write_rs232(43); 			     // listen 29 ID CAN BUS command

write_rs232(250); 			     // timeout 250 mS

rs232_buffer[0]:=read_rs232(100);     // get the response with 100 ms timeout

if read_error then exit               // no response

rs232_buffer[1]:=read_rs232(10);     // get the response with 100 ms timeout

if read_error then exit               // no response

if rs232_buffer[0]<>0

then begin

for i:=1 to length do

begin

RS232_buffer[i]:=read_RS232[10];

end;

end

else ............               // error

| Commande | 44 |
| --- | --- |
| Fonction | CAN BUS Communication 11 bit ID |
| Remarque | This command sends a CAN string and get the response It use the internal header set by the set header command Multiple Frame responses get back max 255 bytes |
| Request | 44 , 8 CAN data field inc. PCI byte Timeout is fixed at 200 mS |
| Pos. Response | 44 , length , data 44 , 0  no data after timeout |
| Neg. Response | N/A |

# Exemple how to get a Mode 1 / PID 0 from a CAN BUS 11 bit in rs232_buffer the 8 data bytes to send are in rs232_buffer. Header is set to 07E0

Rs232_buffer[1]:=02;		  	// PCI

Rs232_buffer[2]:=01;		 	// mode 1

Rs232_buffer[3]:=00;			// PID 0

Rs232_buffer[4]:=0;			       // filled with 0

Rs232_buffer[5]:=0;			       // filled with 0

Rs232_buffer[6]:=0;			       // filled with 0

Rs232_buffer[7]:=0;			       // filled with 0

Rs232_buffer[8]:=0;			       // filled with 0

write_rs232(44); 			     // 11 bit ID communication

for i:=1 to 8 do

begin

write_rs232(RS232_buffer);

end;

rs232_buffer[0]:=read_rs232(100);     // get the response with 100 ms timeout

if read_error then exit               // no response

length:=read_rs232(10);               // get the response length

if read_error then exit               // no response

if rs232_buffer[0]=44

then begin                       // if length <> 0

for i:=1 to length do

begin

RS232_buffer[i]:=read_RS232[10];

end;

end

| Rs232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 2C | Command 44 |
| [1] | 07 | CAN Header |
| [2] | E8 | CAN Header |
| [3] | 06 | PCI |
| [4] | 41 | Response to mode 1 |
| [5] | 00 | PID 00 |
| [6] | XX | Data 1 |
| [7] | Xx | Data 2 |
| [8] | Xx | Data 3 |
| [9] | Xx | Data 4 |
| [10] | 00 | Not used |

| Commande | 45 |
| --- | --- |
| Fonction | CAN BUS Communication 29 bit ID |
| Remarque | This command sends a CAN string and get the response It use the internal header set by the set header command Multiple Frame responses get back max 255 bytes |
| Request | 45 , 8 CAN data field inc. PCI byte Timeout is fixed at 200 mS |
| Pos. Response | 45 , length , data 45 , 0  no data after timeout |
| Neg. Response | N/A |

# Exemple how to get a Mode1 / PID 0 from a CAN BUS 29 bit in rs232_buffer the 8 data

Rs232_buffer[1]:=02;		  	// PCI

Rs232_buffer[2]:=01;		 	// mode 1

Rs232_buffer[3]:=00;			// PID 0

Rs232_buffer[4]:=0;			       // filled with 0

Rs232_buffer[5]:=0;			       // filled with 0

Rs232_buffer[6]:=0;			       // filled with 0

Rs232_buffer[7]:=0;			       // filled with 0

Rs232_buffer[8]:=0;			       // filled with 0

write_rs232(45); 			       // 29 bit ID communication

for i:=1 to 8 do

begin

write_rs232(RS232_buffer);

end;

rs232_buffer[0]:=read_rs232(100);     // get the response with 100 ms timeout

if read_error then exit               // no response

length:=read_rs232(10);               // get the response length

if read_error then exit               // no response

if rs232_buffer[0]=45

then begin

for i:=1 to length do          // length <> 0

begin

RS232_buffer[i]:=read_RS232[10];

end;

end;

| Rs232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 2D | Command 45 |
| [1] | 18 | CAN Header |
| [2] | DA | CAN Header |
| [3] | F1 | Destination address |
| [4] | 10 | Source address (PCM) |
| [5] | 06 | PCI |
| [6] | 41 | Mode 1 |
| [7] | 00 | PID 00 |
| [8] | XX | Data 1 |
| [9] | XX | Data 2 |
| [10] | XX | Data 3 |
| [11] | XX | Data 4 |
| [12] | 00 | Not used |

| Commande | 46 |
| --- | --- |
| Fonction | Set and Init J1939 CAN parameter |
| Remarque | This command sets and initializes J1939 parameter. |
| Request | 46 |
| Pos. Response | 46 , ACK |
| Neg. Response | N/A |

# Exemple

write_rs232(46); 			     // Set and init J1939 parameter

rs232_buffer[0]:=read_rs232(100);     // get the response with 100 ms timeout

if read_error then exit               // no response

rs232_buffer[1]:=read_rs232(10);     // get the response with 100 ms timeout

if read_error then exit               // no response

if rs232_buffer[1]=ACK

then ........			     // ok

| Commande | 47 |
| --- | --- |
| Fonction | J1939 communication |
| Remarque | This command sends a CAN string and get the response It use 29 bit header set with set header command Multiple Frame responses get back max 255 bytes |
| Request | 47 , 8 CAN data Timeout is fixed at 400 mS |
| Pos. Response | 47 , length , data 47 , 0  no data after timeout |
| Neg. Response | N/A |

Please refer to J1939/71 to compose this message.

The command 38 can be used to set an incoming message filter.

| Commande | 50 |
| --- | --- |
| Fonction | Send and get a ALDL8192 string to/from ECU |
| Remarque | This string is sent directly to ECU , checksum is calculated by user . Note that a max of 255 byte can be received. No bytes will be added by mobydic. SO can the user works transparent . |
| Request | 50 , length , data |
| Pos. Response | 50 , length , ALDL response_string if length =0 no response is received from ECU |
| Neg. Response | N/A |

Exemple for command 50 how to send a direct ALDL message to ECU ( clear DTC command )

write_rs232(50); 			    // get ALDL data command

write_rs232(4); 	                 // 4 bytes follow

write_rs232($F4); 	                 //  start frame

write_rs232($56); 	                 //  $52+length

write_rs232($0A); 	                 //  clear DTC command

write_rs232(chksum); 	          //  checksumm allbyte modulo 256/

rs232_buffer[0]:=read_rs232(250);    // wait for 1.byte

if read_error then exit              // no response

if rs232_buffer[0] <> 50 the exit    // false sync.

Length := read_rs232(10);            // get incoming message length

For i:=1 to length do                // if length <> 0

Begin

rs232_buffer[i]:=read_rs232(10); // read length x byte 10 ms timeout

End;

| Commande | 51 |
| --- | --- |
| Fonction | 5 Baud Init on Kw71 ( please use kw1281 commands ) |
| Remarque | 5 Baud init as ISO9141 A keep alive is starting automatically as ACK block exchange. Baudrate is detected and set automatically |
| Request | 51 , ECU_address |
| Pos. Response | 51, length , keyword1 , keyword2 , ID_data |
| Neg. Response | NACK , error_code |

Exemple for command 51 how to init an KW71 BUS

Open_COMM;

clearbuffers;

write_rs232(51); 			    // 5 Baud Init command

write_rs232(01); 			    // ECU address

rs232_buffer[0]:=read_rs232(250);    // wait for 1.byte

if read_error then exit              // no response

if rs232_buffer[0] <> 51 the exit    // false sync.

Length := read_rs232(10);            // get incoming message length

For i:=1 to length do                // if length <> 0

Begin

rs232_buffer[i]:=read_rs232(10); // read length x byte 10 ms timeout

End;

By NACK :

| Error code | description |
| --- | --- |
| 00 | No 0x55 seen ( no response |
| 01 | BUS Busy for more than 5 sec. |

| Commande | 52 |
| --- | --- |
| Fonction | Send a KW71 command to ECU ( please use kw1281 commands ) |
| Remarque | This string is sent directly to ECU , Note that a max of 255 byte can be received. Block counter is handled by mobydic |
| Request | Send 52 and wait for ACK Then send length , data |
| Pos. Response | 52 , length , Kw71 response_string if length =0 no response is received from ECU |
| Neg. Response | N/A |

Exemple for command 52 how to send a  Kw71 message to ECU ( clear DTC command )

write_rs232(52); 			    // get Kw71 data command

rs232_buffer[0]:=read_rs232(250);    // wait for 1.byte

if read_error then exit              // no response

if rs232_buffer[0] <> ACK the exit    // false sync.

write_rs232(1); 	                 //  1 byte follow

write_rs232(05); 	                 //  clear DTC command

Format sent out to ECU by mobydic after this command

| Sent byte | Value | Remarque |
| --- | --- | --- |
| [0] | 03 | Block length |
| [1] | cc | Current block counter |
| [2] | 05 | Code for clear DTC |
| [3] | 03 | ETX |

rs232_buffer[0]:=read_rs232(250);    // wait for 1.byte

if read_error then exit              // no response

if rs232_buffer[0] <> 52 the exit    // false sync.

Length := read_rs232(10);            // get incoming message length

For i:=1 to length do                // if length <> 0

Begin

rs232_buffer[i]:=read_rs232(10); // read length x byte 10 ms timeout

End;

Bytes in receiving Buffer

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 34 | Response to command 52 |
| [1] | 03 | Kw71 message length |
| [2] | cc | Current block counter |
| [3] | 09 | ACK |
| [4] | 03 | ETX |

| Commande | 53 |
| --- | --- |
| Fonction | 5 Baud Init Kw1281 on K-line |
| Remarque | 5 Baud init A keep alive is starting automatically as ACK block exchange. Baudrate is detected and set automatically Repeat the read test till all ID block are received |
| Request | 53 , ECU_address |
| Pos. Response | 53, length , keyword1 , keyword2 , ID_data string |
| Neg. Response | 53 , NACK , error_code |

Exemple for command 53 how to init the engine ECU on KW1281 BUS ( address 0x01 )

Using K line

Open_COMM;

clearbuffers;

write_rs232(53); 			    // 5 Baud Init command

write_rs232(01); 			    // ECU address

rs232_buffer[0]:=read_rs232(250);    // wait for 1.byte

if read_error then exit              // no response

if rs232_buffer[0] <> 53 then exit    // false sync.

Length := read_rs232(10);            // get incoming message length

For i:=1 to length do                // if length <> 0

Begin

rs232_buffer[i]:=read_rs232(10); // read length x byte 10 ms timeout

End;

By NACK :

| Error code | description |
| --- | --- |
| 00 | No 0x55 seen ( no response |
| 01 | BUS Busy for more than 5 sec. |

| Commande | 54 |
| --- | --- |
| Fonction | Communicate via Kw1281 |
| Remarque | Note that a max of 16 bytes as standard message can be received. Block counter is handled by mobydic. User needs only send command code and data if requested. |
| Request | Send 54 and wait for ACK Then send length , data |
| Pos. Response | 54 , length , Kw1281 response string if length =0 no response is received from ECU |
| Neg. Response | N/A |

Exemple for command 54 how to send a  Kw1281 message to ECU ( group reading command )

write_rs232(54); 			    // get Kw1281 data command

rs232_buffer[0]:=read_rs232(250);    // wait for 1.byte

if read_error then exit              // no response

if rs232_buffer[0] <> ACK then exit    // false sync.

write_rs232(2); 	                 //  2 bytes follow

write_rs232($29); 	                 //  group reading command

write_rs232(01); 	                 //  group 1

Format sent out to ECU by mobydic after this command

| Sent byte | Value | Remarque |
| --- | --- | --- |
| [0] | 04 | Block length |
| [1] | CC | Current block counter |
| [2] | 29 | Group reading command |
| [3] | 01 | Group number |
| [4] | 03 | ETX |

rs232_buffer[0]:=read_rs232(250);    // wait for 1.byte

if read_error then exit              // no response

if rs232_buffer[0] <> 54 the exit    // false sync.

Length := read_rs232(10);            // get incoming message length

For i:=1 to length do                // if length <> 0

Begin

rs232_buffer[i]:=read_rs232(10); // read length x byte 10 ms timeout

End;

Bytes in receiving Buffer

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 54 | Response to command 54 |
| [1] | 0F | Kw1281 message length |
| [2] | cc | Current block counter |
| [3] | E7 | Block title |
| [4] | A1 | Standard indicator number |
| [5] | A1_a | 1. value byte a |
| [6] | A1_b | 1. value byte b |
| [7] | A2 | Standard indicator number |
| [8] | A2_a | 2. value byte a |
| [9] | A2_b | 2. value byte b |
| [10] | A3 | Standard indicator number |
| [11] | A3_a | 3. value byte a |
| [12] | A3_b | 3. value byte b |
| [13] | A4 | Standard indicator number |
| [14] | A4_a | 4. value byte a |
| [15] | A4_b | 4. value byte b |
| [16] | 03 | ETX |

Standard indicator number and calculating formula as follow

Type   Conversion                           	Unit

1	a*0.2*b				rpm

2	a*0.002*b				%

3	a*0.002*b				Deg

4	abs(b-127)*0.01*a	ATDC if Value >127, else "BTDC"

5	a*(b-100)*0.1			°C

6	0.001*a*b				V

7	0.01*a*b				km/h

8	0.1*a*b				-

9	(b-127)*0.02*a			Deg

10	b				if b==0 then "COLD", else "WARM"

11	0.0001*a*(b-128)+1		-

12	0.001*a*b				Ohm

13	(b-127)*0.001*a			mm

14	0.005*a*b				bar

15	0.01*a*b				ms

16	???					bitvalue

17	chr(a)	chr(b)				-

18	0.04*a*b				m	bar

19	a*b*0.01				L

20	a*(b-128)/128			%

21	0.001*a*b				V

22	0.001*a*b				ms

23	b/256*a				%

24	0.001*a*b				A

25	(b*1.421)+(a/182)			g/s

26	b-a					C

27	abs(b-128)*0.01*a	ATDC if Value <128, else "BTDC"

28	b-a					-

29	Kennfeld		if b<a then "1.Kennfeld" else "2.Kennfeld"

30	b/12*a				Deg	k/w

31	b/2560*a				°C

32	if b>128 : b-256 else b		-

33	100*b/a(ifa=0then100*b)		%

34	(b-128)*0.01*a			kW

35	0.01*a*b				l/h

36	b*10+a*2560			km

37	???					-

38	(b-128)*0.001*a	Deg		k/w

39	b/256*a				mg/h

40	b*0.1+(25.5*a)-400			A

41	b+a*255				Ah

42	b*0.1+(25.5*a)-400			Kw

43	b*0.1+(25.5*a)			V

44	chr(a)	:	chr(b)	h

45	0.1*a*b/100

46	(a*b-3200)*0.0027			Deg k/w

47	(b-128)*a				ms

48	b+a*255				-

49	(b/4)*a				mg/h

50	(b-128)/(0.01*a),	if a=0	(b-128)/0.01	mBar

51	((b-128)/255)*a			mg/h

52	b*0.02*a-a				Nm

53	(b-128)*1.4222+0.006*a		g/s

54	a*256+b				Count

55	a*b/200				s

56	a*256+b				WSC

57	a*256+b+65536			WSC

58	1.0225*b,	if	b>128	then	1.0225*(256-b)

59	(a*256+b)/32768			-

60	(a*256+b)*0.01			sec

61	(b-128)/a,ifa==0(b-128)		-

62	0.256*a*b				S

63	chr(a) + chr(b) + "?"		-

64	a+b					Ohm

65	0.01*a*(b-127)			mm

66	(a*b)/511.12				V

67	(640*a)+b*2.5			Deg

68	(256*a+b)/7.365			deg/s

69	(256*a+b)*0.3254			Bar

70	(256*a+b)*0.192			m/s^2

Exemple for command 54 how to send a  Kw1281 message to ECU ( read DTC )

write_rs232(54); 			    // get Kw71 data command

rs232_buffer[0]:=read_rs232(250);    // wait for 1.byte

if read_error then exit              // no response

if rs232_buffer[0] <> ACK then exit    // false sync.

write_rs232(1); 	                 //  2 bytes follow

write_rs232(07); 	                 //  Read DTC command

Format sent out to ECU by mobydic after this command

| Sent byte | Value | Remarque |
| --- | --- | --- |
| [0] | 03 | Block length |
| [1] | CC | Current block counter |
| [2] | 07 | Read DTC |
| [3] | 03 | ETX |

rs232_buffer[0]:=read_rs232(250);    // wait for 1.byte

if read_error then exit              // no response

if rs232_buffer[0] <> 54 the exit    // false sync.

Length := read_rs232(10);            // get incoming message length

For i:=1 to length do                // if length <> 0

Begin

rs232_buffer[i]:=read_rs232(10); // read length x byte 10 ms timeout

End;

Bytes in receiving Buffer only 1 fault

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 36 | Response to command 54 |
| [1] | 06 | Kw1281 message length |
| [2] | cc | Current block counter |
| [3] | FC | Block title |
| [4] | F1 | DTC high byte |
| [5] | F1 | DTC low byte |
| [6] | F1 | DTC Status |
| [7] | 03 | ETX |

Exemple for command 54 how to send a  Kw1281 message to ECU ( clear DTC )

write_rs232(54); 			    // get Kw1281 data command

rs232_buffer[0]:=read_rs232(250);    // wait for 1.byte

if read_error then exit              // no response

if rs232_buffer[0] <> ACK then exit    // false sync.

write_rs232(1); 	                 //  1 byte follow

write_rs232(05); 	                 //  clear DTC command

Format sent out to ECU by mobydic after this command

| Sent byte | Value | Remarque |
| --- | --- | --- |
| [0] | 03 | Block length |
| [1] | CC | Current block counter |
| [2] | 05 | Clear DTC title |
| [3] | 03 | ETX |

rs232_buffer[0]:=read_rs232(250);    // wait for 1.byte

if read_error then exit              // no response

if rs232_buffer[0] <> 54 the exit    // false sync.

Length := read_rs232(10);            // get incoming message length

For i:=1 to length do                // if length <> 0

Begin

rs232_buffer[i]:=read_rs232(10); // read length x byte 10 ms timeout

End;

Bytes in receiving Buffer

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 36 | Response to command 54 |
| [1] | 03 | Kw1281 message length |
| [2] | cc | Current block counter |
| [3] | 09 | Block title ACK |
| [7] | 03 | ETX |

| Commande | 55 |
| --- | --- |
| Fonction | Stop auto keep aliving kw1281 |
| Remarque | The ACK block exchange can be stopped by this command. User should take care to do it himself |
| Request | Send 55 |
| Pos. Response | 55,ACK |
| Neg. Response | N/A |

Exemple for command 54 how to send a  Kw1281 message to ECU ( group reading command )

write_rs232(55); 			    // get Kw1281 data command

rs232_buffer[0]:=read_rs232(250);    // wait for 1.byte

if read_error then exit              // no response

rs232_buffer[1]:=read_rs232(250);    // wait for 1.byte

if read_error then exit              // no response

if rs232_buffer[1] <> ACK then exit    // false sync.

| Commande | 57 |
| --- | --- |
| Fonction | 5/200 Baud Init Kw82 on K-line 7 |
| Remarque | 5 / 200 Baud init Baudrate is detected and set automatically |
| Request | 57 , ECU_address , 5/200 5/200 = 0  5 baud 5/200 <> 0  200 baud init |
| Pos. Response | 57, ack |
| Neg. Response | NACK , error_code |

Exemple for command 57 how to init the Airbag ECU on KW82 BUS ( address 0x62 )

Using K line

Open_COMM;

clearbuffers;

write_rs232(57); 			    // 5 Baud Init command

write_rs232($62); 			    // ECU address

write_rs232(1); 			    // 200 baud init

rs232_buffer[0]:=read_rs232(250);    // wait for 1.byte

if read_error then exit              // no response

if rs232_buffer[0] <> 57 then exit   // false sync.

Length := read_rs232(10);            // get incoming message length

For i:=1 to length do                // if length <> 0

Begin

rs232_buffer[i]:=read_rs232(10); // read length x byte 10 ms timeout

End;

By NACK :

| Error code | description |
| --- | --- |
| 00 | No 0x55 seen ( no response |
| 01 | Sync error |

| Commande | 58 |
| --- | --- |
| Fonction | Communicate via Kw81/82 |
| Remarque | Note that a max of 255 byte can be received. User sends block title and data if necessary. The data come continously out |
| Request | Send 58 and wait for ACK Then send length , data |
| Pos. Response | 58 , length , Kw82 response string if length =0 no response is received from ECU |
| Neg. Response | N/A |

Exemple for command 58 how to send a  Kw81/82 message to ECU ( read ID )

write_rs232(58); 			    // get Kw82 data command

rs232_buffer[0]:=read_rs232(250);    // wait for 1.byte

if read_error then exit              // no response

if rs232_buffer[0] <> ACK then exit    // false sync.

write_rs232(4); 	                 //  4 bytes follow

write_rs232(2); 	                 //  real block length

write_rs232($10); 	                 //  read snapshoot

write_rs232(chksum); 	                 //  2 bytes chksum

write_rs232(chksum); 	                 //

Format sent out to ECU by mobydic after this command

| Sent byte | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Block length |
| [1] | 10 | Read ECU ID |
| [2].[3] | chksum | ChecksumHB,,LB |

rs232_buffer[0]:=read_rs232(250);    // wait for 1.byte

if read_error then exit              // no response

if rs232_buffer[0] <> 58 the exit    // false sync.

Length := read_rs232(10);            // get incoming message length

For i:=1 to length do                // if length <> 0

Begin

rs232_buffer[i]:=read_rs232(10); // read length x byte 10 ms timeout

end;

Bytes in receiving Buffer

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 3A | Response to command 58 |
| [1] | L | Total Message length |
| [2] | N | Kw82 message length |
| [3] | A0 | Block title |
| [3..N] | -- | Data |
| [N+1],[N+2] | CHKSUM | Checksum |

MULTIPLE BUFFER HANDLING ON CAN BUS ( extended command set )

MOByDic 2700 has 1 Transmit object and 3 receive buffers object that can be programmed separately. So the user can set 3 incoming filter and 3 masks on CAN Bus and communicate with 3 CAN-ECU simultanously. The main receive channel ( channel 1 )  is handled with commands 30-47 . Setting a baudrate is provided in these extended commands that allow the user to set a special Baudrate

| Commande | 60 |
| --- | --- |
| Fonction | Set Incoming Filter and Mask on CAN BUS 11 ID channel 1 |
| Remarque | Default setting is $7E8 ( EOBD control modul I) |
| Request | 60 , ECU_Address_HB,ECU_address_LB, Mask_HB , Mask_LB |
| Pos. Response | 60 , ACK ( 06 ) |
| Neg. Response | N/A |

Exemple for command 60 how setting the mask in mObydic to TCM on CAN Bus

Open_COMM;

clearbuffers;

write_rs232(60); 			     // send set address command

write_rs232($07); 			     // send TCM address to mOByDic write_rs232($E9); 			     // send TCM address to mOByDic write_rs232($00); 			     // no mask

write_rs232($00); 			     // no mask

rs232_buffer[0]:=read_rs232(100);     // get the response with 100 ms timeout

if read_error then exit               // no response

rs232_buffer[1]:=read_rs232(10);      // get the response with 10 ms timeout

if read_error then exit               // no response

if rs232_buffer[1] = ACK

then ..................           // handle

| Commande | 61 |
| --- | --- |
| Fonction | Set Incoming Filter and Mask on CAN BUS 11 ID channel 2 |
| Remarque | Default setting is $7E9 ( EOBD control modul II) |
| Request | 61 , ECU_Address_HB,ECU_address_LB, Mask_HB , Mask_LB |
| Pos. Response | 61 , ACK ( 06 ) |
| Neg. Response | N/A |

Exemple for command 61 how setting the mask in mObydic to PCM on CAN Bus

Open_COMM;

clearbuffers;

write_rs232(61); 			     // send set address command

write_rs232($07); 			     // send pCM address to mOByDic write_rs232($E8); 			     // send pCM address to mOByDic write_rs232($00); 			     // no mask

write_rs232($00); 			     // no mask

rs232_buffer[0]:=read_rs232(100);     // get the response with 100 ms timeout

if read_error then exit               // no response

rs232_buffer[1]:=read_rs232(10);      // get the response with 10 ms timeout

if read_error then exit               // no response

if rs232_buffer[1] = ACK

then ..................           // handle

| Commande | 62 |
| --- | --- |
| Fonction | Set Incoming Filter and Mask on CAN BUS 11 ID channel 3 |
| Remarque | Default setting is $7EA ( EOBD control modul III) |
| Request | 62 , ECU_Address_HB,ECU_address_LB, Mask_HB , Mask_LB |
| Pos. Response | 62 , ACK ( 06 ) |
| Neg. Response | N/A |

Exemple for command 62 how setting the mask in mObydic to ABS on CAN Bus

Open_COMM;

clearbuffers;

write_rs232(62); 			     // send set address command

write_rs232($07); 			     // send ABS address to mOByDic write_rs232($28); 			     // send ABS address to mOByDic write_rs232($07); 			     // accept only message from this address

write_rs232($28); 			     //

rs232_buffer[0]:=read_rs232(100);     // get the response with 100 ms timeout

if read_error then exit               // no response

rs232_buffer[1]:=read_rs232(10);      // get the response with 10 ms timeout

if read_error then exit               // no response

if rs232_buffer[1] = ACK

then ..................           // handle

| Commande | 63 |
| --- | --- |
| Fonction | Read Incoming Filter and Mask on CAN BUS 11 ID channel 1 |
| Remarque | The user can check the set mask and filter |
| Request | 63 |
| Pos. Response | 63 , ECU_Address_HB,ECU_address_LB, Mask_HB , Mask_LB |
| Neg. Response | N/A |

Exemple for command 63 how reading the filter and mask in mObydic on CAN Bus

Open_COMM;

clearbuffers;

write_rs232(63); 			     // send set address command

rs232_buffer[0]:=read_rs232(100);     // get the response with 100 ms timeout

if read_error then exit               // no response

rs232_buffer[1]:=read_rs232(10);      // get the response with 10 ms timeout

rs232_buffer[2]:=read_rs232(10);      // get the response with 10 ms timeout

rs232_buffer[3]:=read_rs232(10);      // get the response with 10 ms timeout

rs232_buffer[4]:=read_rs232(10);      // get the response with 10 ms timeout

| Commande | 64 |
| --- | --- |
| Fonction | Read Incoming Filter and Mask on CAN BUS 11 ID channel 2 |
| Remarque | The user can check the set mask and filter |
| Request | 64 |
| Pos. Response | 64 , ECU_Address_HB,ECU_address_LB, Mask_HB , Mask_LB |
| Neg. Response | N/A |

Exemple for command 64 how reading the filter and mask in mObydic on CAN Bus

Open_COMM;

clearbuffers;

write_rs232(64); 			     // send set address command

rs232_buffer[0]:=read_rs232(100);     // get the response with 100 ms timeout

if read_error then exit               // no response

rs232_buffer[1]:=read_rs232(10);      // get the response with 10 ms timeout

rs232_buffer[2]:=read_rs232(10);      // get the response with 10 ms timeout

rs232_buffer[3]:=read_rs232(10);      // get the response with 10 ms timeout

rs232_buffer[4]:=read_rs232(10);      // get the response with 10 ms timeout

| Commande | 65 |
| --- | --- |
| Fonction | Read Incoming Filter and Mask on CAN BUS 11 ID channel 3 |
| Remarque | The user can check the set mask and filter |
| Request | 65 |
| Pos. Response | 65 , ECU_Address_HB,ECU_address_LB, Mask_HB , Mask_LB |
| Neg. Response | N/A |

Exemple for command 64 how reading the filter and mask in mObydic on CAN Bus

Open_COMM;

clearbuffers;

write_rs232(65); 			     // send set address command

rs232_buffer[0]:=read_rs232(100);     // get the response with 100 ms timeout

if read_error then exit               // no response

rs232_buffer[1]:=read_rs232(10);      // get the response with 10 ms timeout

rs232_buffer[2]:=read_rs232(10);      // get the response with 10 ms timeout

rs232_buffer[3]:=read_rs232(10);      // get the response with 10 ms timeout

rs232_buffer[4]:=read_rs232(10);      // get the response with 10 ms timeout

| Commande | 66 |
| --- | --- |
| Fonction | Set Baudrate to CAN BUS |
| Remarque |  |
| Request | 66 , CANBT1,CANBT2,CANBT3 |
| Pos. Response | 66,ACK |
| Neg. Response | N/A |

To set the baudrate register of mobydic2700 please refer the datasheets of ATMEL t89c51cc02 CAN register and Baudrate calculator. The frequency is 16 Mhz in x2 mode

| Commande | 67 |
| --- | --- |
| Fonction | Send a CAN BUS 11 bit ID message |
| Remarque | This command sends a CAN string It use the internal header set by the set header command Multiple Frame responses get back max 255 bytes |
| Request | 67 , 8 CAN data field inc. PCI byte Timeout is fixed at 200 mS |
| Pos. Response | 67,ACK |
| Neg. Response | 67,NACK transmit error after 3 retries |

write_rs232(67); 			      // send set address command

Rs232_buffer[1]:=02;		  	// PCI

Rs232_buffer[2]:=01;		 	// mode 1

Rs232_buffer[3]:=00;			// PID 0

Rs232_buffer[4]:=0;			       // filled with 0

Rs232_buffer[5]:=0;			       // filled with 0

Rs232_buffer[6]:=0;			       // filled with 0

Rs232_buffer[7]:=0;			       // filled with 0

Rs232_buffer[8]:=0;			       // filled with 0

Send_CAN_Buffer;                        // send 8 bytes

rs232_buffer[0]:=read_rs232(100);       // get the response with 100 ms timeout

if read_error then exit                 // no response

rs232_buffer[1]:=read_rs232(10);        // get the response with 10 ms timeout

if read_error then exit                 // no response

if rs232_buffer[1]=ACK then             // handle

| Commande | 68 |
| --- | --- |
| Fonction | Read a CAN BUS 11 bit ID message |
| Remarque | This command reads one of 3 CAN receive buffer |
| Request | 68 , Channel ( 1..3 ) |
| Pos. Response | 68, response string |
| Neg. Response | N/A |

write_rs232(68); 			     // Read CAN data command

write_rs232(1); 			     // from 1.channel

Read_CAN_Channel;                     // read following byte into buffer

Read channel byte ( response to an EOBD service 1 PID 0 request )

| Rs232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 44 | Command 68 |
| [1] | 07 | CAN Header |
| [2] | E8 | CAN Header |
| [3] | 06 | PCI |
| [4] | 41 | Response to mode 1 |
| [5] | 00 | PID 00 |
| [6] | XX | Data 1 |
| [7] | Xx | Data 2 |
| [8] | Xx | Data 3 |
| [9] | Xx | Data 4 |
| [10] | 00 | Not used |

| Commande | 70 |
| --- | --- |
| Fonction | Set Incoming Filter and Mask on CAN BUS 29 ID channel 1 |
| Remarque | Default setting is $18 DA F1 10 ( EOBD control modul ) |
| Request | 70 , ECU_A1,ECU_A2, ECU_A3,ECU_A4, Mask_1 , Mask_2 , Mask_3 , Mask_4 |
| Pos. Response | 70 , ACK ( 06 ) |
| Neg. Response | N/A |

Exemple for command 70 how setting the mask in mObydic on CAN Bus

Open_COMM;

clearbuffers;

write_rs232(70); 			     // send set address command

write_rs232($18); 			     // send TCM address to mOByDic write_rs232($DA); 			     // send TCM address to mOByDic write_rs232($F1); 			     // send TCM address to mOByDic write_rs232($18); 			     // send TCM address to mOByDic write_rs232($00); 			     // no mask

write_rs232($00); 			     // no mask

write_rs232($00); 			     // no mask

write_rs232($00); 			     // no mask

rs232_buffer[0]:=read_rs232(100);     // get the response with 100 ms timeout

if read_error then exit               // no response

rs232_buffer[1]:=read_rs232(10);      // get the response with 10 ms timeout

if read_error then exit               // no response

if rs232_buffer[1] = ACK

then ..................           // handle

| Commande | 71 |
| --- | --- |
| Fonction | Set Incoming Filter and Mask on CAN BUS 29 ID channel 2 |
| Remarque | Default setting is $18 DA F1 10 ( EOBD control modul ) |
| Request | 71 , ECU_A1,ECU_A2, ECU_A3,ECU_A4, Mask_1 , Mask_2 , Mask_3 , Mask_4 |
| Pos. Response | 71 , ACK ( 06 ) |
| Neg. Response | N/A |

Exemple for command 71 how setting the mask in mObydic on CAN Bus

Open_COMM;

clearbuffers;

write_rs232(71); 			     // send set address command

write_rs232($18); 			     // send TCM address to mOByDic write_rs232($DA); 			     // send TCM address to mOByDic write_rs232($F1); 			     // send TCM address to mOByDic write_rs232($18); 			     // send TCM address to mOByDic write_rs232($00); 			     // no mask

write_rs232($00); 			     // no mask

write_rs232($00); 			     // no mask

write_rs232($00); 			     // no mask

rs232_buffer[0]:=read_rs232(100);     // get the response with 100 ms timeout

if read_error then exit               // no response

rs232_buffer[1]:=read_rs232(10);      // get the response with 10 ms timeout

if read_error then exit               // no response

if rs232_buffer[1] = ACK

then ..................           // handle

| Commande | 72 |
| --- | --- |
| Fonction | Set Incoming Filter and Mask on CAN BUS 29 ID channel 3 |
| Remarque | Default setting is $18 DA F1 10 ( EOBD control modul ) |
| Request | 72 , ECU_A1,ECU_A2, ECU_A3,ECU_A4, Mask_1 , Mask_2 , Mask_3 , Mask_4 |
| Pos. Response | 72 , ACK ( 06 ) |
| Neg. Response | N/A |

Exemple for command 72 how setting the mask in mObydic on CAN Bus

Open_COMM;

clearbuffers;

write_rs232(72); 			     // send set address command

write_rs232($18); 			     // send TCM address to mOByDic write_rs232($DA); 			     // send TCM address to mOByDic write_rs232($F1); 			     // send TCM address to mOByDic write_rs232($18); 			     // send TCM address to mOByDic write_rs232($00); 			     // no mask

write_rs232($00); 			     // no mask

write_rs232($00); 			     // no mask

write_rs232($00); 			     // no mask

rs232_buffer[0]:=read_rs232(100);     // get the response with 100 ms timeout

if read_error then exit               // no response

rs232_buffer[1]:=read_rs232(10);      // get the response with 10 ms timeout

if read_error then exit               // no response

if rs232_buffer[1] = ACK

then ..................           // handle

| Commande | 73 |
| --- | --- |
| Fonction | Read Incoming Filter and Mask on CAN BUS 29 ID channel 1 |
| Remarque |  |
| Request | 73 |
| Pos. Response | 73 , ECU_A1,ECU_A2, ECU_A3,ECU_A4, Mask_1 , Mask_2 , Mask_3 , Mask_4 |
| Neg. Response | N/A |

Exemple for command 73 how reading the mask in mObydic on CAN Bus

Open_COMM;

clearbuffers;

write_rs232(73); 			     // send set address command

rs232_buffer[0]:=read_rs232(100);     // get the response with 100 ms timeout

if read_error then exit               // no response

rs232_buffer[1]:=read_rs232(10);      // get the response with 10 ms timeout

rs232_buffer[2]:=read_rs232(10);      // get the response with 10 ms timeout

rs232_buffer[3]:=read_rs232(10);      // get the response with 10 ms timeout

rs232_buffer[4]:=read_rs232(10);      // get the response with 10 ms timeout

rs232_buffer[5]:=read_rs232(10);      // get the response with 10 ms timeout

rs232_buffer[6]:=read_rs232(10);      // get the response with 10 ms timeout

rs232_buffer[7]:=read_rs232(10);      // get the response with 10 ms timeout

rs232_buffer[8]:=read_rs232(10);      // get the response with 10 ms timeout

| Commande | 74 |
| --- | --- |
| Fonction | Read Incoming Filter and Mask on CAN BUS 29 ID channel 2 |
| Remarque |  |
| Request | 74 |
| Pos. Response | 74 , ECU_A1,ECU_A2, ECU_A3,ECU_A4, Mask_1 , Mask_2 , Mask_3 , Mask_4 |
| Neg. Response | N/A |

Exemple for command 74 how reading the mask in mObydic on CAN Bus

Open_COMM;

clearbuffers;

write_rs232(74); 			     // send set address command

rs232_buffer[0]:=read_rs232(100);     // get the response with 100 ms timeout

if read_error then exit               // no response

rs232_buffer[1]:=read_rs232(10);      // get the response with 10 ms timeout

rs232_buffer[2]:=read_rs232(10);      // get the response with 10 ms timeout

rs232_buffer[3]:=read_rs232(10);      // get the response with 10 ms timeout

rs232_buffer[4]:=read_rs232(10);      // get the response with 10 ms timeout

rs232_buffer[5]:=read_rs232(10);      // get the response with 10 ms timeout

rs232_buffer[6]:=read_rs232(10);      // get the response with 10 ms timeout

rs232_buffer[7]:=read_rs232(10);      // get the response with 10 ms timeout

rs232_buffer[8]:=read_rs232(10);      // get the response with 10 ms timeout

| Commande | 75 |
| --- | --- |
| Fonction | Read Incoming Filter and Mask on CAN BUS 29 ID channel 3 |
| Remarque |  |
| Request | 75 |
| Pos. Response | 75 , ECU_A1,ECU_A2, ECU_A3,ECU_A4, Mask_1 , Mask_2 , Mask_3 , Mask_4 |
| Neg. Response | N/A |

Exemple for command 75 how reading the mask in mObydic on CAN Bus

Open_COMM;

clearbuffers;

write_rs232(75); 			     // send set address command

rs232_buffer[0]:=read_rs232(100);     // get the response with 100 ms timeout

if read_error then exit               // no response

rs232_buffer[1]:=read_rs232(10);      // get the response with 10 ms timeout

rs232_buffer[2]:=read_rs232(10);      // get the response with 10 ms timeout

rs232_buffer[3]:=read_rs232(10);      // get the response with 10 ms timeout

rs232_buffer[4]:=read_rs232(10);      // get the response with 10 ms timeout

rs232_buffer[5]:=read_rs232(10);      // get the response with 10 ms timeout

rs232_buffer[6]:=read_rs232(10);      // get the response with 10 ms timeout

rs232_buffer[7]:=read_rs232(10);      // get the response with 10 ms timeout

rs232_buffer[8]:=read_rs232(10);      // get the response with 10 ms timeout

| Commande | 77 |
| --- | --- |
| Fonction | Send a CAN BUS 29 bit ID message |
| Remarque | This command sends a CAN string It use the internal header set by the set header command Multiple Frame responses get back max 255 bytes |
| Request | 77 , 8 CAN data field inc. PCI byte Timeout is fixed at 200 mS |
| Pos. Response | 77,ACK |
| Neg. Response | 77,NACK transmit error after 3 retries |

write_rs232(77); 			      // send set address command

Rs232_buffer[1]:=02;		  	// PCI

Rs232_buffer[2]:=01;		 	// mode 1

Rs232_buffer[3]:=00;			// PID 0

Rs232_buffer[4]:=0;			       // filled with 0

Rs232_buffer[5]:=0;			       // filled with 0

Rs232_buffer[6]:=0;			       // filled with 0

Rs232_buffer[7]:=0;			       // filled with 0

Rs232_buffer[8]:=0;			       // filled with 0

Send_CAN_Buffer;                        // send 8 bytes

rs232_buffer[0]:=read_rs232(100);       // get the response with 100 ms timeout

if read_error then exit                 // no response

rs232_buffer[1]:=read_rs232(10);        // get the response with 10 ms timeout

if read_error then exit                 // no response

if rs232_buffer[1]=ACK then             // handle

| Commande | 78 |
| --- | --- |
| Fonction | CAN BUS Communication 29 bit ID |
| Remarque | This command gets the response It use the internal header set by the set header command Multiple Frame responses get back max 255 bytes |
| Request | 78 , Channel ( 1..3 ) |
| Pos. Response | 78 , length , data 78 , 0  no data after timeout |
| Neg. Response | N/A |

# Exemple how to get a Mode1 / PID 0 from a CAN BUS 29 bit in rs232_buffer the 8 data

write_rs232(78); 			       // read 29 bit ID communication

write_rs232(01); 			       // read 1.CAN channel

rs232_buffer[0]:=read_rs232(100);     // get the response with 100 ms timeout

if read_error then exit               // no response

length:=read_rs232(10);               // get the response length

if read_error then exit               // no response

if rs232_buffer[0]=78

then begin

for i:=1 to length do          // length <> 0

begin

RS232_buffer[i]:=read_RS232[10];

end;

end;

| Rs232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 4E | Command 78 |
| [1] | 18 | CAN Header |
| [2] | DA | CAN Header |
| [3] | F1 | Destination address |
| [4] | 10 | Source address (PCM) |
| [5] | 06 | PCI |
| [6] | 41 | Mode 1 |
| [7] | 00 | PID 00 |
| [8] | XX | Data 1 |
| [9] | XX | Data 2 |
| [10] | XX | Data 3 |
| [11] | XX | Data 4 |
| [12] | 00 | Not used |

# GENERIC MODE

Mobydic has a generic mode implemented. After succesfully connection as EOBD this mode allows to send only EOBD mode 1..9 data field without knowing the found protocol. Software developer doesnt need to handle protocol specific messages. The generic mode use the fonction 2 like in 2600.

The generic messages are functional messages, which means the external test equipment will request data without knowledge of which ECU(s) on the vehicle will respond. In some vehicles, multiple ECUs may respond with the information requested.  For ISO 9141-2 and KWP2000 interfaces, Data Link Layer response time requirements are max 50 mS .

For SAE J1850 network interfaces, the on-board systems responds to a request within 100mS of a request or a previous response message. With multiple response messages possible from a single request message, this allows as much time as is necessary for all ECUs to access the data link and transmit their response message

For CAN bus systems based on ISO 15765-4, the ECUs of the on-board system respond to a request message within 50mS.

For SAE J1850 and ISO 9141-2 interfaces, there will be no reject message to a functional request message if the request is not supported by the ECU. This prevents response messages from all ECUs that do not support a mode or a specific data value.

For ISO 14230-4 interfaces, there will be a response message to every request message either positive  or negative. In order to avoid unnecessary communication the ECU  which does  not support a functionally requested PID is permitted to not send a negative response message.

Some modes are supported by a vehicle, but data may not always be available when requested. For modes 05 and 06, if the test has not been run since test results were cleared, or for mode 02 if freeze frame data has not been stored, For CAN The ECUs which do not support the functional request message dont send any response message. If an ECU does not support any of the PIDs requested it is not allowed to send a negative response messageThe ECUs which do not currently have the requested data available respond with a negative response message (22) - ConditionsNotCorrect

For SAE J1850 and ISO 9141-2 interfaces each unique diagnostic message specified in this standard is a fixed length, although not all messages are the same length. For ISO 14230-4 interfaces, the message length is always determined by the length information included in the first byte of the header.

The maximum message length is specified in ISO 15765-2  is limited to seven 7 data bytes.The first 3 bytes of all diagnostic messages are the header bytes. For SAE J1850 and ISO 9141-2 interfaces the value of the first header byte is dependant on the bit rate of the data link and the type of message . The second header byte has a value that depends on the type of message, either a request or a response.For ISO 14230-4 interfaces, the value of the first header byte indicates the addressing mode (physical/functional) and the length of the data field. The second header byte is the address of the receiver of the message. The third header byte for all interfaces is the physical address of the sender of the message. The external test equipment has the address F1.

| Header bytes |  |  |  |  |  | Data bytes |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Priority/Type |  | Target address |  | Source address (hex) |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | CRC |
| J1850-VPWM and ISO9141-2 Request |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 68 |  | 6A |  | F1 |  | Maximum 7 data bytes |  |  |  |  |  |  | CKS |
| J1850-VPWM and ISO9141-2 Response |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 48 |  | 6B |  | ECU addr |  | Maximum 7 data bytes |  |  |  |  |  |  | CKS |
| Kwp2000 Request |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11LL LLLLb |  | 33 |  | F1 |  | Maximum 7 data bytes |  |  |  |  |  |  | CKS |
| Kwp2000 Response |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10LL LLLLb |  | F1 |  | addr |  | Maximum 7 data bytes |  |  |  |  |  |  | CKS |
| J1850-PWM Request |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 61 |  | 6A |  | F1 |  | Maximum 7 data bytes |  |  |  |  |  |  | CRC |
| J1850-PWM Response |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 41 |  | 6B |  | addr |  | Maximum 7 data bytes |  |  |  |  |  |  | CRC |
| CAN BUS Request 11 bit |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 07 |  |  | E0..E7 |  |  | PCI | Maximum 7 data bytes |  |  |  |  |  | CRC |
| CAN BUS Response 11 bit |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 07 |  |  | E8-EF |  |  | PCI | Maximum 7 data bytes |  |  |  |  |  | CRC |
| CAN BUS Request 29 bit |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 | DB |  | 33 |  | F1 | PCI | Maximum 7 data bytes |  |  |  |  |  | CRC |
| CAN BUS Response 29 bit |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 | DA |  | F1 |  | addr | PCI | Maximum 7 data bytes |  |  |  |  |  | CRC |

| Commande | 02 |
| --- | --- |
| Fonction | OBDII/EOBD generic commands |
| Request | 02 |
| Pos. Response | <06> |
|  | THEN SEND |
| MODE 1 | <01> <PID> |
| MODE 2 | <02> <PID> |
| MODE 3 | <03> |
| MODE 4 | <04> |
| MODE 5 | <05> <TID> <O2SNO> |
| MODE 6 | <06> <TID> |
| MODE 7 | <07> |
| MODE 8 | <08> <TID> <DataA><DataB><DataC><DataD><DataE> |
| MODE 9 | <09> <INFO_Type> |
| Response String | <02> <length> <response string> length = 0  no valid data |
| Neg. Response | N/A |

# Response string

02 , length , data

Software handshaking

write_rs232(02); 			    // Send generic access command 02

rs232_buffer[0]:=read_rs232(250);    // wait for ACK

if read_error then exit              // no response

if rs232_buffer[0] <> 06 then exit   // not ACK

Now send mode and requested PID ( exemple PID = 00 )

write_rs232(MODE); 			    //

write_rs232(PARAMETER); 	          // PID , TID ,INFOTYPE

# MODE 1

It allows access to current emission-related data values, including analogue inputs and outputs, digital inputs and outputs, and system status information. The request for information includes a parameter identification PID.

All data values returned for sensor readings will be actual readings, not default or substitute values used by the system because of a fault with that sensor.

PID 00 is a bit-encoded PID that indicates, for each ECU, which PIDs that ECU

supports.

Handshaking in generic mode

write_rs232(02); 			    // Send generic access command 02

rs232_buffer[0]:=read_rs232(250);    // wait for ACK

if read_error then exit              // no response

if rs232_buffer[0] <> 06 then exit   // not ACK

Now send mode and requested PID ( exemple PID = 00 )

write_rs232(01); 			    // Mode 01

write_rs232(00); 			    // PID  00

Now get response 02 and response length

rs232_buffer[0]:=read_rs232(250);    // wait for 02

if read_error then exit              // no response

if rs232_buffer[0] <> 02 then exit   // false sync.

rs232_buffer[1]:=read_rs232(10);     // wait for 1.byte

if read_error then exit              // no response

if rs232_buffer[1] = 0

then ...........              // length = 00 handle error

Length := rs232_buffer[1];           // get incoming message length

For i:=1 to length do                // if length <> 0

Begin

rs232_buffer[i]:=read_rs232(10); // read length x byte 10 ms timeout

if read_error then ......        // handle error

End;

Bytes in receiving Buffer for ISO9141-2  single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 0A | message length |
| [2] | 48 | Header1 |
| [3] | 6B | Header2 |
| [4] | 10 | Responding ECU address |
| [5] | 41 | Mode 01 response |
| [6] | 00 | Requested PID confirmation ( PID=00) |
| [7] | XX | Implemented PID1..8 |
| [8] | XX | Implemented PID9..16 |
| [9] | XX | Implemented PID17..24 |
| [10] | XX | Implemented PID25..32 |
| [11] | Chksum | Checksum of ISO message |

Bytes in receiving Buffer for ISO9141-2  2 ECUs are responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 14 | Message length |
| [2] | 48 | Header1 |
| [3] | 6B | Header2 |
| [4] | 10 | Responding ECU PCM address |
| [5] | 41 | Mode 01 response |
| [6] | 00 | Requested PID confirmation ( PID=00) |
| [7] | XX | Implemented PID1..8 |
| [8] | XX | Implemented PID9..16 |
| [9] | XX | Implemented PID17..24 |
| [10] | XX | Implemented PID25..32 |
| [11] | Chksum | Checksum of ISO1 message |
| [12] | 48 | Header1 |
| [13] | 6B | Header2 |
| [14] | 18 | Responding ECU TCM address |
| [15] | 41 | Mode 01 response |
| [16] | 00 | Requested PID confirmation ( PID=00) |
| [17] | XX | Implemented PID1..8 |
| [18] | XX | Implemented PID9..16 |
| [19] | XX | Implemented PID17..24 |
| [20] | XX | Implemented PID25..32 |
| [21] | Chksum | Checksum of ISO2 message |

Bytes in receiving Buffer for J1850-VPW  single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 0A | message length |
| [2] | 48 | Header1 |
| [3] | 6B | Header2 |
| [4] | 10 | Responding ECU address |
| [5] | 41 | Mode 01 response |
| [6] | 00 | Requested PID confirmation ( PID=00) |
| [7] | XX | Implemented PID1..8 |
| [8] | XX | Implemented PID9..16 |
| [9] | XX | Implemented PID17..24 |
| [10] | XX | Implemented PID25..32 |
| [11] | CRC | CRC of J1850 message |

Bytes in receiving Buffer for J1850-VPW   2 ECUs are responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 14 | Message length |
| [2] | 48 | Header1 |
| [3] | 6B | Header2 |
| [4] | 10 | Responding ECU PCM address |
| [5] | 41 | Mode 01 response |
| [6] | 00 | Requested PID confirmation ( PID=00) |
| [7] | XX | Implemented PID1..8 |
| [8] | XX | Implemented PID9..16 |
| [9] | XX | Implemented PID17..24 |
| [10] | XX | Implemented PID25..32 |
| [11] | CRC | CRC of J1850-1 message |
| [12] | 48 | Header1 |
| [13] | 6B | Header2 |
| [14] | 18 | Responding ECU TCM address |
| [15] | 41 | Mode 01 response |
| [16] | 00 | Requested PID confirmation ( PID=00) |
| [17] | XX | Implemented PID1..8 |
| [18] | XX | Implemented PID9..16 |
| [19] | XX | Implemented PID17..24 |
| [20] | XX | Implemented PID25..32 |
| [21] | CRC | CRC of J1850-2 message |

Bytes in receiving Buffer for J1850-PWM  single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 0A | message length |
| [2] | 41 | Header1 |
| [3] | 6B | Header2 |
| [4] | 10 | Responding ECU address |
| [5] | 41 | Mode 01 response |
| [6] | 00 | Requested PID confirmation ( PID=00) |
| [7] | XX | Implemented PID1..8 |
| [8] | XX | Implemented PID9..16 |
| [9] | XX | Implemented PID17..24 |
| [10] | XX | Implemented PID25..32 |
| [11] | CRC | CRC of J1850 message |

Bytes in receiving Buffer for J1850 - PWM   2 ECUs are responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 14 | Message length |
| [2] | 41 | Header1 |
| [3] | 6B | Header2 |
| [4] | 10 | Responding ECU PCM address |
| [5] | 41 | Mode 01 response |
| [6] | 00 | Requested PID confirmation ( PID=00) |
| [7] | XX | Implemented PID1..8 |
| [8] | XX | Implemented PID9..16 |
| [9] | XX | Implemented PID17..24 |
| [10] | XX | Implemented PID25..32 |
| [11] | CRC | CRC of J1850-1 message |
| [12] | 41 | Header1 |
| [13] | 6B | Header2 |
| [14] | 18 | Responding ECU TCM address |
| [15] | 41 | Mode 01 response |
| [16] | 00 | Requested PID confirmation ( PID=00) |
| [17] | XX | Implemented PID1..8 |
| [18] | XX | Implemented PID9..16 |
| [19] | XX | Implemented PID17..24 |
| [20] | XX | Implemented PID25..32 |
| [21] | CRC | CRC of J1850-2 message |

Bytes in receiving Buffer for KWP2000  single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 0A | message length |
| [2] | 86 | Header1 |
| [3] | F1 | Header2 |
| [4] | 10 | Responding ECU address |
| [5] | 41 | Mode 01 response |
| [6] | 00 | Requested PID confirmation ( PID=00) |
| [7] | XX | Implemented PID1..8 |
| [8] | XX | Implemented PID9..16 |
| [9] | XX | Implemented PID17..24 |
| [10] | XX | Implemented PID25..32 |
| [11] | Chksum | Checksum of KWP message |

Bytes in receiving Buffer for KWP2000  2 ECUs are responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 14 | Message length |
| [2] | 86 | Header1 |
| [3] | F1 | Header2 |
| [4] | 10 | Responding ECU PCM address |
| [5] | 41 | Mode 01 response |
| [6] | 00 | Requested PID confirmation ( PID=00) |
| [7] | XX | Implemented PID1..8 |
| [8] | XX | Implemented PID9..16 |
| [9] | XX | Implemented PID17..24 |
| [10] | XX | Implemented PID25..32 |
| [11] | Chksum | Checksum of KWP2000-1 message |
| [12] | 86 | Header1 |
| [13] | F1 | Header2 |
| [14] | 18 | Responding ECU TCM address |
| [15] | 41 | Mode 01 response |
| [16] | 00 | Requested PID confirmation ( PID=00) |
| [17] | XX | Implemented PID1..8 |
| [18] | XX | Implemented PID9..16 |
| [19] | XX | Implemented PID17..24 |
| [20] | XX | Implemented PID25..32 |
| [21] | Chksum | Checksum of KWP2000-2 message |

Bytes in receiving Buffer for CAN – 11 single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 0A | message length |
| [2] | 07 | Header1 |
| [3] | E8 | Header2 |
| [4] | 06 | PCI |
| [5] | 41 | Mode 01 response |
| [6] | 00 | Requested PID confirmation ( PID=00) |
| [7] | XX | Implemented PID1..8 |
| [8] | XX | Implemented PID9..16 |
| [9] | XX | Implemented PID17..24 |
| [10] | XX | Implemented PID25..32 |
| [11] | 00 | Not used bytes must be filled with 00 |

Bytes in receiving Buffer for CAN-11  2 ECUs are responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 14 | Message length |
| [2] | 07 | Header1 |
| [3] | E8 | Header2 |
| [4] | 06 | PCI |
| [5] | 41 | Mode 01 response |
| [6] | 00 | Requested PID confirmation ( PID=00) |
| [7] | XX | Implemented PID1..8 |
| [8] | XX | Implemented PID9..16 |
| [9] | XX | Implemented PID17..24 |
| [10] | XX | Implemented PID25..32 |
| [11] | 00 | Not used bytes must be filled with 00 |
| [12] | 07 | Header1 |
| [13] | E9 | Header2 |
| [14] | 06 | PCI |
| [15] | 41 | Mode 01 response |
| [16] | 00 | Requested PID confirmation ( PID=00) |
| [17] | XX | Implemented PID1..8 |
| [18] | XX | Implemented PID9..16 |
| [19] | XX | Implemented PID17..24 |
| [20] | XX | Implemented PID25..32 |
| [21] | 00 | Not used bytes must be filled with 00 |

Bytes in receiving Buffer for CAN – 29 single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 0C | message length |
| [2] | 18 | Header1 |
| [3] | DA | Header2 |
| [4] | F1 | Destination address |
| [5] | 10 | Source address |
| [6] | 06 | PCI |
| [7] | 41 | Mode 01 response |
| [8] | 00 | Requested PID confirmation ( PID=00) |
| [9] | XX | Implemented PID1..8 |
| [10] | XX | Implemented PID9..16 |
| [11] | XX | Implemented PID17..24 |
| [12] | XX | Implemented PID25..32 |
| [13] | 00 | Not used bytes must be filled with 00 |

Bytes in receiving Buffer for CAN – 29  2 ECUs are responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 18 | message length |
| [2] | 18 | Header1 |
| [3] | DA | Header2 |
| [4] | F1 | Destination address |
| [5] | 10 | Source address |
| [6] | 06 | PCI |
| [7] | 41 | Mode 01 response |
| [8] | 00 | Requested PID confirmation ( PID=00) |
| [9] | XX | Implemented PID1..8 |
| [10] | XX | Implemented PID9..16 |
| [11] | XX | Implemented PID17..24 |
| [12] | XX | Implemented PID25..32 |
| [13] | 00 | Not used bytes must be filled with 00 |
| [14] | 18 | Header1 |
| [15] | DA | Header2 |
| [16] | F1 | Destination address |
| [17] | 18 | Source address |
| [18] | 06 | PCI |
| [19] | 41 | Mode 01 response |
| [20] | 00 | Requested PID confirmation ( PID=00) |
| [21] | XX | Implemented PID1..8 |
| [22] | XX | Implemented PID9..16 |
| [23] | XX | Implemented PID17..24 |
| [24] | XX | Implemented PID25..32 |
| [25] | 00 | Not used bytes must be filled with 00 |

## MODE 2

It allows access to emission-related data values in a freeze frame. The request message includes a parameter identification PID)

PID 02 indicates the DTC that caused the freeze frame data to be stored. If freeze frame data is not stored in the ECU, the system reports 00 00 as the DTC. The frame number byte 00 is reserved for the mandated freeze frame data.

Exemple tell how to get the DTC number caused freeze frame storage

Handshaking in generic mode

write_rs232(02); 			    // Send generic access command 02

rs232_buffer[0]:=read_rs232(250);    // wait for ACK

if read_error then exit              // no response

if rs232_buffer[0] <> 06 then exit   // not ACK

Now send mode and requested PID ( exemple PID = 00 )

write_rs232(02); 			    // Mode 02

write_rs232(02); 			    // PID02 get DTC caused freeze frame storage

Now get response 02 and response length

rs232_buffer[0]:=read_rs232(250);    // wait for 02

if read_error then exit              // no response

if rs232_buffer[0] <> 02 then exit   // false sync.

rs232_buffer[1]:=read_rs232(10);     // wait for 1.byte

if read_error then exit              // no response

if rs232_buffer[1] = 0

then ...........              // length = 00 handle error

Length := rs232_buffer[1];           // get incoming message length

For i:=1 to length do

Begin

rs232_buffer[i]:=read_rs232(10); // read length x byte 10 ms timeout

if read_error then ......        // handle error

End;

Bytes in receiving Buffer for ISO9141-2  single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 09 | message length |
| [2] | 48 | Header1 |
| [3] | 6B | Header2 |
| [4] | 10 | Responding ECU address |
| [5] | 42 | Mode 02 response |
| [6] | 02 | Requested PID confirmation ( PID=02) |
| [7] | 00 | Frame #0 |
| [8] | XX | DTC_High |
| [9] | XX | DTC_low |
| [10] | Chksum | Checksum of ISO message |

Bytes in receiving Buffer for ISO9141-2  2 ECUs are responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 12 | Message length |
| [2] | 48 | Header1 |
| [3] | 6B | Header2 |
| [4] | 10 | Responding ECU PCM address |
| [5] | 42 | Mode 02 response |
| [6] | 02 | Requested PID confirmation ( PID=02) |
| [7] | 00 | #frame 0 |
| [8] | XX | DTC_high |
| [9] | XX | DTC_low |
| [10] | Chksum | Checksum of ISO1 message |
| [11] | 48 | Header1 |
| [12] | 6B | Header2 |
| [13] | 18 | Responding ECU TCM address |
| [14] | 42 | Mode 02 response |
| [15] | 02 | Requested PID confirmation ( PID=02) |
| [16] | 00 | #frame 0 |
| [17] | XX | DTC_high |
| [18] | XX | DTC_low |
| [19] | Chksum | Checksum of ISO2 message |

Bytes in receiving Buffer for J1850-VPW  single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 09 | message length |
| [2] | 48 | Header1 |
| [3] | 6B | Header2 |
| [4] | 10 | Responding ECU address |
| [5] | 42 | Mode 02 response |
| [6] | 02 | Requested PID confirmation ( PID=02) |
| [7] | 00 | Frame #0 |
| [8] | XX | DTC_High |
| [9] | XX | DTC_low |
| [10] | CRC | CRC of j1850 message |

Bytes in receiving Buffer for J1850-VPW  2 ECUs are responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 12 | Message length |
| [2] | 48 | Header1 |
| [3] | 6B | Header2 |
| [4] | 10 | Responding ECU PCM address |
| [5] | 42 | Mode 02 response |
| [6] | 02 | Requested PID confirmation ( PID=02) |
| [7] | 00 | #frame 0 |
| [8] | XX | DTC_high |
| [9] | XX | DTC_low |
| [10] | CRC | CRC of 1.message |
| [11] | 48 | Header1 |
| [12] | 6B | Header2 |
| [13] | 18 | Responding ECU TCM address |
| [14] | 42 | Mode 02 response |
| [15] | 02 | Requested PID confirmation ( PID=02) |
| [16] | 00 | #frame 0 |
| [17] | XX | DTC_high |
| [18] | XX | DTC_low |
| [19] | CRC | CRC of 2.message |

Bytes in receiving Buffer for J1850-PWM  single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 09 | message length |
| [2] | 41 | Header1 |
| [3] | 6B | Header2 |
| [4] | 10 | Responding ECU address |
| [5] | 42 | Mode 02 response |
| [6] | 02 | Requested PID confirmation ( PID=02) |
| [7] | 00 | Frame #0 |
| [8] | XX | DTC_High |
| [9] | XX | DTC_low |
| [10] | CRC | CRC of j1850 message |

Bytes in receiving Buffer for J1850-PWM  2 ECUs are responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 12 | Message length |
| [2] | 41 | Header1 |
| [3] | 6B | Header2 |
| [4] | 10 | Responding ECU PCM address |
| [5] | 42 | Mode 02 response |
| [6] | 02 | Requested PID confirmation ( PID=02) |
| [7] | 00 | #frame 0 |
| [8] | XX | DTC_high |
| [9] | XX | DTC_low |
| [10] | CRC | CRC of 1.message |
| [11] | 41 | Header1 |
| [12] | 6B | Header2 |
| [13] | 18 | Responding ECU TCM address |
| [14] | 42 | Mode 02 response |
| [15] | 02 | Requested PID confirmation ( PID=02) |
| [16] | 00 | #frame 0 |
| [17] | XX | DTC_high |
| [18] | XX | DTC_low |
| [19] | CRC | CRC of 2.message |

Bytes in receiving Buffer for KWP2000  single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 09 | message length |
| [2] | 85 | Header1 |
| [3] | F1 | Header2 |
| [4] | 10 | Responding ECU address |
| [5] | 42 | Mode 02 response |
| [6] | 02 | Requested PID confirmation ( PID=02) |
| [7] | 00 | #frame 0 |
| [8] | XX | DTC_high |
| [9] | XX | DTC_low |
| [10] | Chksum | Checksum of KWP message |

Bytes in receiving Buffer for KWP2000  2 ECUs are responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 12 | Message length |
| [2] | 85 | Header1 |
| [3] | F1 | Header2 |
| [4] | 10 | Responding ECU PCM address |
| [5] | 42 | Mode 02 response |
| [6] | 02 | Requested PID confirmation ( PID=02) |
| [7] | 00 | #frame 0 |
| [8] | XX | DTC_high |
| [9] | XX | DTC_low |
| [10] | Chksum | Checksum of KWP2000-1 message |
| [11] | 85 | Header1 |
| [12] | F1 | Header2 |
| [13] | 18 | Responding ECU TCM address |
| [14] | 42 | Mode 01 response |
| [15] | 02 | Requested PID confirmation ( PID=02) |
| [16] | 00 | #frame 0 |
| [17] | XX | DTC_high |
| [18] | XX | DTC_low |
| [19] | Chksum | Checksum of KWP2000-2 message |

Bytes in receiving Buffer for CAN – 11 single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 0A | message length |
| [2] | 07 | Header1 |
| [3] | E8 | Header2 |
| [4] | 05 | PCI |
| [5] | 42 | Mode 02 response |
| [6] | 02 | Requested PID confirmation ( PID=02) |
| [7] | 00 | Frame #0 |
| [8] | XX | DTC_High |
| [9] | XX | DTC_low |
| [10] | 00 | Not used bytes must be filled with 00 |
| [11] | 00 | Not used bytes must be filled with 00 |

Bytes in receiving Buffer for CAN-11  2 ECUs are responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 14 | Message length |
| [2] | 07 | Header1 |
| [3] | E8 | Header2 |
| [4] | 05 | PCI |
| [5] | 42 | Mode 02 response |
| [6] | 02 | Requested PID confirmation ( PID=02) |
| [7] | 00 | Frame #0 |
| [8] | XX | DTC_High |
| [9] | XX | DTC_low |
| [10] | 00 | Not used bytes must be filled with 00 |
| [11] | 00 | Not used bytes must be filled with 00 |
| [12] | 07 | Header1 |
| [13] | E9 | Header2 |
| [14] | 05 | PCI |
| [15] | 42 | Mode 02 response |
| [16] | 02 | Requested PID confirmation ( PID=02) |
| [17] | 00 | Frame #0 |
| [18] | XX | DTC_High |
| [19] | XX | DTC_low |
| [20] | 00 | Not used bytes must be filled with 00 |
| [21] | 00 | Not used bytes must be filled with 00 |

Bytes in receiving Buffer for CAN – 29 single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 0C | message length |
| [2] | 18 | Header1 |
| [3] | DA | Header2 |
| [4] | F1 | Destination address |
| [5] | 10 | Source address |
| [6] | 05 | PCI |
| [7] | 42 | Mode 02 response |
| [8] | 02 | Requested PID confirmation ( PID=02) |
| [9] | 00 | Frame #0 |
| [10] | XX | DTC_High |
| [11] | XX | DTC_low |
| [12] | 00 | Not used bytes must be filled with 00 |
| [13] | 00 | Not used bytes must be filled with 00 |

Bytes in receiving Buffer for CAN – 29  2 ECUs are responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 18 | message length |
| [2] | 18 | Header1 |
| [3] | DA | Header2 |
| [4] | F1 | Destination address |
| [5] | 10 | Source address |
| [6] | 05 | PCI |
| [7] | 42 | Mode 02 response |
| [8] | 02 | Requested PID confirmation ( PID=02) |
| [9] | 00 | Frame #0 |
| [10] | XX | DTC_High |
| [11] | XX | DTC_low |
| [12] | 00 | Not used bytes must be filled with 00 |
| [13] | 00 | Not used bytes must be filled with 00 |
| [14] | 18 | Header1 |
| [15] | DA | Header2 |
| [16] | F1 | Destination address |
| [17] | 18 | Source address |
| [18] | 05 | PCI |
| [19] | 42 | Mode 02 response |
| [20] | 02 | Requested PID confirmation ( PID=02) |
| [21] | 00 | Frame #0 |
| [22] | XX | DTC_High |
| [23] | XX | DTC_low |
| [24] | 00 | Not used bytes must be filled with 00 |
| [25] | 00 | Not used bytes must be filled with 00 |

# MODE 3

It allows to obtain emission-related DTCs. First send a mode 01, PID 01 request to get the number of emission-related  DTCs from all ECUs Each ECU that has a DTC(s) stored will respond with a message that includes the number of stored codes to be reported. If an ECU idoes not have stored DTCs,responds with a message indicating zero DTCs are stored.

Send a Mode 03 request for all emission-related  DTCs. Each ECU responds with one or more messages, each containing up to three (3) DTCs. If no emission-related  DTCs are stored in the ECU, then the ECU may not respond to this request.  For more than 3 DTC , multiple responses come

Exemple tell how to get the DTCs

Handshaking in generic mode

write_rs232(02); 			    // Send generic access command 02

rs232_buffer[0]:=read_rs232(250);    // wait for ACK

if read_error then exit              // no response

if rs232_buffer[0] <> 06 then exit   // not ACK

Now send mode 03

write_rs232(03); 			    // Mode 03

Now get response 02 and response length

rs232_buffer[0]:=read_rs232(250);    // wait for 02

if read_error then exit              // no response

if rs232_buffer[0] <> 02 then exit   // false sync.

rs232_buffer[1]:=read_rs232(10);     // wait for 1.byte

if read_error then exit              // no response

if rs232_buffer[1] = 0

then ...........              // length = 00 handle error

Length := rs232_buffer[1];           // get incoming message length

For i:=1 to length do

Begin

rs232_buffer[i]:=read_rs232(10); // read length x byte 10 ms timeout

if read_error then ......        // handle error

End;

Bytes in receiving Buffer for ISO9141-2  single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 07 | message length |
| [2] | 48 | Header1 |
| [3] | 6B | Header2 |
| [4] | 10 | Responding ECU address |
| [5] | 43 | Mode 03 response |
| [6] | XX | DTC_High |
| [7] | XX | DTC_low |
| [8] | Chksum | Checksum of ISO message |

Bytes in receiving Buffer for ISO9141-2  2 ECUs are responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 0E | Message length |
| [2] | 48 | Header1 |
| [3] | 6B | Header2 |
| [4] | 10 | Responding ECU PCM address |
| [5] | 43 | Mode 03 response |
| [6] | XX | DTC_high |
| [7] | XX | DTC_low |
| [8] | Chksum | Checksum of ISO1 message |
| [9] | 48 | Header1 |
| [10] | 6B | Header2 |
| [11] | 18 | Responding ECU TCM address |
| [12] | 43 | Mode 03 response |
| [13] | XX | DTC_high |
| [14] | XX | DTC_low |
| [15] | Chksum | Checksum of ISO2 message |

Bytes in receiving Buffer for J1850-VPW  single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 07 | message length |
| [2] | 48 | Header1 |
| [3] | 6B | Header2 |
| [4] | 10 | Responding ECU address |
| [5] | 43 | Mode 03 response |
| [6] | XX | DTC_High |
| [7] | XX | DTC_low |
| [8] | CRC | CRC of j1850 message |

Bytes in receiving Buffer for J1850-VPW  2 ECUs are responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 0E | Message length |
| [2] | 48 | Header1 |
| [3] | 6B | Header2 |
| [4] | 10 | Responding ECU PCM address |
| [5] | 43 | Mode 03 response |
| [6] | XX | DTC_high |
| [7] | XX | DTC_low |
| [8] | CRC | CRC of 1.message |
| [9] | 48 | Header1 |
| [10] | 6B | Header2 |
| [11] | 18 | Responding ECU TCM address |
| [12] | 43 | Mode 03 response |
| [13] | XX | DTC_high |
| [14] | XX | DTC_low |
| [15] | CRC | CRC of 2.message |

Bytes in receiving Buffer for J1850-PWM  single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 07 | message length |
| [2] | 41 | Header1 |
| [3] | 6B | Header2 |
| [4] | 10 | Responding ECU address |
| [5] | 43 | Mode 03 response |
| [6] | XX | DTC_High |
| [7] | XX | DTC_low |
| [8] | CRC | CRC of j1850 message |

Bytes in receiving Buffer for J1850-PWM  2 ECUs are responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 0E | Message length |
| [2] | 41 | Header1 |
| [3] | 6B | Header2 |
| [4] | 10 | Responding ECU PCM address |
| [5] | 43 | Mode 03 response |
| [6] | XX | DTC_high |
| [7] | XX | DTC_low |
| [8] | CRC | CRC of 1.message |
| [9] | 41 | Header1 |
| [10] | 6B | Header2 |
| [11] | 18 | Responding ECU TCM address |
| [12] | 43 | Mode 03 response |
| [13] | XX | DTC_high |
| [14] | XX | DTC_low |
| [15] | CRC | CRC of 2.message |

Bytes in receiving Buffer for KWP2000  single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 07 | message length |
| [2] | 83 | Header1 |
| [3] | F1 | Header2 |
| [4] | 10 | Responding ECU address |
| [5] | 43 | Mode 03 response |
| [6] | XX | DTC_high |
| [7] | XX | DTC_low |
| [8] | Chksum | Checksum of KWP message |

Bytes in receiving Buffer for KWP2000  2 ECUs are responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 0E | Message length |
| [2] | 83 | Header1 |
| [3] | F1 | Header2 |
| [4] | 10 | Responding ECU PCM address |
| [5] | 43 | Mode 03 response |
| [6] | XX | DTC_high |
| [7] | XX | DTC_low |
| [8] | Chksum | Checksum of KWP2000-1 message |
| [9] | 83 | Header1 |
| [10] | F1 | Header2 |
| [11] | 18 | Responding ECU TCM address |
| [12] | 43 | Mode 03 response |
| [13] | XX | DTC_high |
| [14] | XX | DTC_low |
| [15] | Chksum | Checksum of KWP2000-2 message |

Bytes in receiving Buffer for CAN – 11 single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 0A | message length |
| [2] | 07 | Header1 |
| [3] | E8 | Header2 |
| [4] | 04 | PCI |
| [5] | 43 | Mode 03 response |
| [6] | 01 | # of DTC |
| [7] | XX | DTC_High |
| [8] | XX | DTC_low |
| [9] | 00 | Not used bytes must be filled with 00 |
| [10] | 00 | Not used bytes must be filled with 00 |
| [11] | 00 | Not used bytes must be filled with 00 |

Bytes in receiving Buffer for CAN-11  2 ECUs are responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 14 | Message length |
| [2] | 07 | Header1 |
| [3] | E8 | Header2 |
| [4] | 04 | PCI |
| [5] | 43 | Mode 03 response |
| [6] | 01 | # of DTC |
| [7] | XX | DTC_High |
| [8] | XX | DTC_low |
| [9] | 00 | Not used bytes must be filled with 00 |
| [10] | 00 | Not used bytes must be filled with 00 |
| [11] | 00 | Not used bytes must be filled with 00 |
| [12] | 07 | Header1 |
| [13] | E9 | Header2 |
| [14] | 04 | PCI |
| [15] | 43 | Mode 03 response |
| [16] | 02 | # of DTC |
| [17] | XX | DTC_High |
| [18] | XX | DTC_low |
| [19] | 00 | Not used bytes must be filled with 00 |
| [20] | 00 | Not used bytes must be filled with 00 |
| [21] | 00 | Not used bytes must be filled with 00 |

Bytes in receiving Buffer for CAN – 29 single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 0C | message length |
| [2] | 18 | Header1 |
| [3] | DA | Header2 |
| [4] | F1 | Destination address |
| [5] | 10 | Source address |
| [6] | 04 | PCI |
| [7] | 43 | Mode 03 response |
| [8] | 01 | # of DTC |
| [9] | XX | DTC_High |
| [10] | XX | DTC_low |
| [11] | 00 | Not used bytes must be filled with 00 |
| [12] | 00 | Not used bytes must be filled with 00 |
| [13] | 00 | Not used bytes must be filled with 00 |

Bytes in receiving Buffer for CAN – 29  2 ECUs are responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 18 | message length |
| [2] | 18 | Header1 |
| [3] | DA | Header2 |
| [4] | F1 | Destination address |
| [5] | 10 | Source address |
| [6] | 04 | PCI |
| [7] | 43 | Mode 03 response |
| [8] | 01 | # of DTC |
| [9] | XX | DTC_High |
| [10] | XX | DTC_low |
| [11] | 00 | Not used bytes must be filled with 00 |
| [12] | 00 | Not used bytes must be filled with 00 |
| [13] | 00 | Not used bytes must be filled with 00 |
| [14] | 18 | Header1 |
| [15] | DA | Header2 |
| [16] | F1 | Destination address |
| [17] | 18 | Source address |
| [18] | 04 | PCI |
| [19] | 43 | Mode 03 response |
| [20] | 01 | # of DTC |
| [21] | XX | DTC_High |
| [22] | XX | DTC_low |
| [23] | 00 | Not used bytes must be filled with 00 |
| [24] | 00 | Not used bytes must be filled with 00 |
| [25] | 00 | Not used bytes must be filled with 00 |

MODE 4

It allows to clear all emission-related diagnostic information.

Exemple tell how to clear the DTCs

Handshaking in generic mode

write_rs232(02); 			    // Send generic access command 02

rs232_buffer[0]:=read_rs232(250);    // wait for ACK

if read_error then exit              // no response

if rs232_buffer[0] <> 06 then exit   // not ACK

Now send mode 04

write_rs232(04); 			    // Mode 04

Now get response 02 and response length

rs232_buffer[0]:=read_rs232(250);    // wait for 02

if read_error then exit              // no response

if rs232_buffer[0] <> 02 then exit   // false sync.

rs232_buffer[1]:=read_rs232(10);     // wait for 1.byte

if read_error then exit              // no response

if rs232_buffer[1] = 0

then ...........              // length = 00 handle error

Length := rs232_buffer[1];           // get incoming message length

For i:=1 to length do

Begin

rs232_buffer[i]:=read_rs232(10); // read length x byte 10 ms timeout

if read_error then ......        // handle error

End;

Bytes in receiving Buffer for ISO9141-2  single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 05 | message length |
| [2] | 48 | Header1 |
| [3] | 6B | Header2 |
| [4] | 10 | Responding ECU address |
| [5] | 44 | Mode 04 response |
| [6] | Chksum | Checksum of ISO message |

Bytes in receiving Buffer for ISO9141-2  2 ECUs are responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 0A | Message length |
| [2] | 48 | Header1 |
| [3] | 6B | Header2 |
| [4] | 10 | Responding ECU PCM address |
| [5] | 44 | Mode 04 response |
| [6] | Chksum | Checksum of ISO1 message |
| [7] | 48 | Header1 |
| [8] | 6B | Header2 |
| [9] | 18 | Responding ECU TCM address |
| [10] | 44 | Mode 04 response |
| [11] | Chksum | Checksum of ISO2 message |

Bytes in receiving Buffer for J1850-VPW  single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 05 | message length |
| [2] | 48 | Header1 |
| [3] | 6B | Header2 |
| [4] | 10 | Responding ECU address |
| [5] | 44 | Mode 04 response |
| [6] | CRC | CRC of j1850 message |

Bytes in receiving Buffer for J1850-VPW  2 ECUs are responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 0A | Message length |
| [2] | 48 | Header1 |
| [3] | 6B | Header2 |
| [4] | 10 | Responding ECU PCM address |
| [5] | 44 | Mode 04 response |
| [6] | CRC | CRC of 1.message |
| [7] | 48 | Header1 |
| [8] | 6B | Header2 |
| [9] | 18 | Responding ECU TCM address |
| [10] | 44 | Mode 04 response |
| [11] | CRC | CRC of 2.message |

Bytes in receiving Buffer for J1850-PWM  single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 07 | message length |
| [2] | 41 | Header1 |
| [3] | 6B | Header2 |
| [4] | 10 | Responding ECU address |
| [5] | 44 | Mode 03 response |
| [6] | CRC | CRC of j1850 message |

Bytes in receiving Buffer for J1850-PWM  2 ECUs are responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 0A | Message length |
| [2] | 41 | Header1 |
| [3] | 6B | Header2 |
| [4] | 10 | Responding ECU PCM address |
| [5] | 44 | Mode 04 response |
| [6] | CRC | CRC of 1.message |
| [7] | 41 | Header1 |
| [8] | 6B | Header2 |
| [9] | 18 | Responding ECU TCM address |
| [10] | 44 | Mode 04 response |
| [11] | CRC | CRC of 2.message |

Bytes in receiving Buffer for KWP2000  single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 07 | message length |
| [2] | 81 | Header1 |
| [3] | F1 | Header2 |
| [4] | 10 | Responding ECU address |
| [5] | 44 | Mode 04 response |
| [6] | Chksum | Checksum of KWP message |

Bytes in receiving Buffer for KWP2000  2 ECUs are responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 0E | Message length |
| [2] | 81 | Header1 |
| [3] | F1 | Header2 |
| [4] | 10 | Responding ECU PCM address |
| [5] | 44 | Mode 04 response |
| [6] | Chksum | Checksum of KWP2000-1 message |
| [7] | 81 | Header1 |
| [8] | F1 | Header2 |
| [9] | 18 | Responding ECU TCM address |
| [10] | 44 | Mode 04 response |
| [11] | Chksum | Checksum of KWP2000-2 message |

Bytes in receiving Buffer for CAN – 11 single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 0A | message length |
| [2] | 07 | Header1 |
| [3] | E8 | Header2 |
| [4] | 01 | PCI |
| [5] | 44 | Mode 04 response |
| [6] | 00 | Not used bytes must be filled with 00 |
| [7] | 00 | Not used bytes must be filled with 00 |
| [8] | 00 | Not used bytes must be filled with 00 |
| [9] | 00 | Not used bytes must be filled with 00 |
| [10] | 00 | Not used bytes must be filled with 00 |
| [11] | 00 | Not used bytes must be filled with 00 |

Bytes in receiving Buffer for CAN-11  2 ECUs are responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 14 | Message length |
| [2] | 07 | Header1 |
| [3] | E8 | Header2 |
| [4] | 01 | PCI |
| [5] | 44 | Mode 04 response |
| [6] | 00 | Not used bytes must be filled with 00 |
| [7] | 00 | Not used bytes must be filled with 00 |
| [8] | 00 | Not used bytes must be filled with 00 |
| [9] | 00 | Not used bytes must be filled with 00 |
| [10] | 00 | Not used bytes must be filled with 00 |
| [11] | 00 | Not used bytes must be filled with 00 |
| [12] | 07 | Header1 |
| [13] | E9 | Header2 |
| [14] | 01 | PCI |
| [15] | 44 | Mode 04 response |
| [16] | 00 | Not used bytes must be filled with 00 |
| [17] | 00 | Not used bytes must be filled with 00 |
| [18] | 00 | Not used bytes must be filled with 00 |
| [19] | 00 | Not used bytes must be filled with 00 |
| [20] | 00 | Not used bytes must be filled with 00 |
| [21] | 00 | Not used bytes must be filled with 00 |

Bytes in receiving Buffer for CAN – 29 single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 0C | message length |
| [2] | 18 | Header1 |
| [3] | DA | Header2 |
| [4] | F1 | Destination address |
| [5] | 10 | Source address |
| [6] | 01 | PCI |
| [7] | 44 | Mode 04 response |
| [8] | 00 | Not used bytes must be filled with 00 |
| [9] | 00 | Not used bytes must be filled with 00 |
| [10] | 00 | Not used bytes must be filled with 00 |
| [11] | 00 | Not used bytes must be filled with 00 |
| [12] | 00 | Not used bytes must be filled with 00 |
| [13] | 00 | Not used bytes must be filled with 00 |

Bytes in receiving Buffer for CAN – 29  2 ECUs are responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 18 | message length |
| [2] | 18 | Header1 |
| [3] | DA | Header2 |
| [4] | F1 | Destination address |
| [5] | 10 | Source address |
| [6] | 01 | PCI |
| [7] | 44 | Mode 04 response |
| [8] | 00 | Not used bytes must be filled with 00 |
| [9] | 00 | Not used bytes must be filled with 00 |
| [10] | 00 | Not used bytes must be filled with 00 |
| [11] | 00 | Not used bytes must be filled with 00 |
| [12] | 00 | Not used bytes must be filled with 00 |
| [13] | 00 | Not used bytes must be filled with 00 |
| [14] | 18 | Header1 |
| [15] | DA | Header2 |
| [16] | F1 | Destination address |
| [17] | 18 | Source address |
| [18] | 01 | PCI |
| [19] | 44 | Mode 04 response |
| [20] | 00 | Not used bytes must be filled with 00 |
| [21] | 00 | Not used bytes must be filled with 00 |
| [22] | 00 | Not used bytes must be filled with 00 |
| [23] | 00 | Not used bytes must be filled with 00 |
| [24] | 00 | Not used bytes must be filled with 00 |
| [25] | 00 | Not used bytes must be filled with 00 |

MODE 5

It allows access to the on-board oxygen sensor monitoring test results. The same information may be obtained by the use of this mode . Test results are requested by Test ID. Mode 5 is not supported by CAN

The ECU indicates which Test IDs are supported in the same concept as used for PID support in Mode 01 and 02 . the following exemple shows how to get this results from only ECU1

Handshaking in generic mode

write_rs232(02); 			    // Send generic access command 02

rs232_buffer[0]:=read_rs232(250);    // wait for ACK

if read_error then exit              // no response

if rs232_buffer[0] <> 06 then exit   // not ACK

Now send mode 05

write_rs232(05); 			    // Mode 05

write_rs232(05); 			    // TID see ISO15031-5

write_rs232(01);                     // Bank 1 sensor 1 see ISO15031-5

Now get response 02 and response length

rs232_buffer[0]:=read_rs232(250);    // wait for 02

if read_error then exit              // no response

if rs232_buffer[0] <> 02 then exit   // false sync.

rs232_buffer[1]:=read_rs232(10);     // wait for 1.byte

if read_error then exit              // no response

if rs232_buffer[1] = 0

then ...........              // length = 00 handle error

Length := rs232_buffer[1];           // get incoming message length

For i:=1 to length do

Begin

rs232_buffer[i]:=read_rs232(10); // read length x byte 10 ms timeout

if read_error then ......        // handle error

End;

Bytes in receiving Buffer for ISO9141-2  single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 0A | message length |
| [2] | 48 | Header1 |
| [3] | 6B | Header2 |
| [4] | 10 | Responding ECU address |
| [5] | 45 | Mode 05 response |
| [6] | 05 | TID 5 |
| [7] | 01 | 1. O2 sensor |
| [8] | XX | TESTVAL |
| [9] | XX | MINLIMIT |
| [10] | XX | MAXLIMIT |
| [11] | Chksum | Checksum of ISO message |

Bytes in receiving Buffer for J1850-VPW  single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 0A | message length |
| [2] | 48 | Header1 |
| [3] | 6B | Header2 |
| [4] | 10 | Responding ECU address |
| [5] | 45 | Mode 05 response |
| [6] | 05 | TID 5 |
| [7] | 01 | 1. O2 sensor |
| [8] | XX | TESTVAL |
| [9] | XX | MINLIMIT |
| [10] | XX | MAXLIMIT |
| [11] | CRC | CRC |

Bytes in receiving Buffer for J1850-PWM  single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 0A | message length |
| [2] | 41 | Header1 |
| [3] | 6B | Header2 |
| [4] | 10 | Responding ECU address |
| [5] | 45 | Mode 05 response |
| [6] | 05 | TID 5 |
| [7] | 01 | 1. O2 sensor |
| [8] | XX | TESTVAL |
| [9] | XX | MINLIMIT |
| [10] | XX | MAXLIMIT |
| [11] | CRC | CRC |

Bytes in receiving Buffer for KWP2000  single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 0A | message length |
| [2] | 86 | Header1 |
| [3] | F1 | Header2 |
| [4] | 10 | Responding ECU address |
| [5] | 45 | Mode 05 response |
| [6] | 05 | TID 5 |
| [7] | 01 | 1. O2 sensor |
| [8] | XX | TESTVAL |
| [9] | XX | MINLIMIT |
| [10] | XX | MAXLIMIT |
| [11] | Chksum | Checksum of KWP message |

MODE 6

The purpose of this service is to allow access to the results for on-board diagnostic monitoring tests of specific components/systems that are not continuously monitored. Examples are catalyst monitoring and the evaporative system monitoring.

The ECU indicates which Test IDs are supported in the same concept as used for PID support in Mode 01 and 02 . Handling is as mode 5

MODE 7

It allows to obtain pending diagnostic trouble codes for emission-related components/systems that are tested or continuously monitored during normal driving conditions. It is required for all DTCs and is independent of Mode 03.

After clearing diagnostic information, by reporting test results after a single driving cycle. If the test failed during the driving cycle, the DTC associated with that test will be reported. Test results reported by this mode do not necessarily indicate a faulty component/system. If test results indicate a failure after additional driving, then the MIL will be illuminated and a DTC will be set .

Exemple tell how to get the DTCs

Handshaking in generic mode

write_rs232(02); 			    // Send generic access command 02

rs232_buffer[0]:=read_rs232(250);    // wait for ACK

if read_error then exit              // no response

if rs232_buffer[0] <> 06 then exit   // not ACK

Now send mode 07

write_rs232(07); 			    // Mode 07

Now get response  02 and response length

rs232_buffer[0]:=read_rs232(250);    // wait for 02

if read_error then exit              // no response

if rs232_buffer[0] <> 02 then exit   // false sync.

rs232_buffer[1]:=read_rs232(10);     // wait for 1.byte

if read_error then exit              // no response

if rs232_buffer[1] = 0

then ...........              // length = 00 handle error

Length := rs232_buffer[1];           // get incoming message length

For i:=1 to length do

Begin

rs232_buffer[i]:=read_rs232(10); // read length x byte 10 ms timeout

if read_error then ......        // handle error

End;

Bytes in receiving Buffer for ISO9141-2  single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 07 | message length |
| [2] | 48 | Header1 |
| [3] | 6B | Header2 |
| [4] | 10 | Responding ECU address |
| [5] | 47 | Mode 07 response |
| [6] | XX | DTC_High |
| [7] | XX | DTC_low |
| [8] | Chksum | Checksum of ISO message |

Bytes in receiving Buffer for J1850-VPW  single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 07 | message length |
| [2] | 48 | Header1 |
| [3] | 6B | Header2 |
| [4] | 10 | Responding ECU address |
| [5] | 47 | Mode 07 response |
| [6] | XX | DTC_High |
| [7] | XX | DTC_low |
| [8] | CRC | CRC of j1850 message |

Bytes in receiving Buffer for J1850-PWM  single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 07 | message length |
| [2] | 41 | Header1 |
| [3] | 6B | Header2 |
| [4] | 10 | Responding ECU address |
| [5] | 47 | Mode 07 response |
| [6] | XX | DTC_High |
| [7] | XX | DTC_low |
| [8] | CRC | CRC of j1850 message |

Bytes in receiving Buffer for KWP2000  single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 07 | message length |
| [2] | 83 | Header1 |
| [3] | F1 | Header2 |
| [4] | 10 | Responding ECU address |
| [5] | 47 | Mode 07 response |
| [6] | XX | DTC_high |
| [7] | XX | DTC_low |
| [8] | Chksum | Checksum of KWP message |

Bytes in receiving Buffer for CAN – 11 single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 0A | message length |
| [2] | 07 | Header1 |
| [3] | E8 | Header2 |
| [4] | 04 | PCI |
| [5] | 47 | Mode 07 response |
| [6] | 01 | # of DTC |
| [7] | XX | DTC_High |
| [8] | XX | DTC_low |
| [9] | 00 | Not used bytes must be filled with 00 |
| [10] | 00 | Not used bytes must be filled with 00 |
| [11] | 00 | Not used bytes must be filled with 00 |

Bytes in receiving Buffer for CAN – 29 single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 0C | message length |
| [2] | 18 | Header1 |
| [3] | DA | Header2 |
| [4] | F1 | Destination address |
| [5] | 10 | Source address |
| [6] | 04 | PCI |
| [7] | 47 | Mode 07 response |
| [8] | 01 | # of DTC |
| [9] | XX | DTC_High |
| [10] | XX | DTC_low |
| [11] | 00 | Not used bytes must be filled with 00 |
| [12] | 00 | Not used bytes must be filled with 00 |
| [13] | 00 | Not used bytes must be filled with 00 |

MODE 8

It allows to control the operation of an on-board system, test or component.

ECU  indicates which Test IDs are supported in the same concept as used for PID support in Mode 01 and 02 .

Exemple tell how to do an evaporative system leak test

Handshaking in generic mode

write_rs232(02); 			    // Send generic access command 02

rs232_buffer[0]:=read_rs232(250);    // wait for ACK

if read_error then exit              // no response

if rs232_buffer[0] <> 06 then exit   // not ACK

Now send mode 08

write_rs232(08); 			    // Mode 08

write_rs232(01); 			    // Test 01

Now get response  02 and response length

rs232_buffer[0]:=read_rs232(250);    // wait for 02

if read_error then exit              // no response

if rs232_buffer[0] <> 02 then exit   // false sync.

rs232_buffer[1]:=read_rs232(10);     // wait for 1.byte

if read_error then exit              // no response

if rs232_buffer[1] = 0

then ...........              // length = 00 handle error

Length := rs232_buffer[1];           // get incoming message length

For i:=1 to length do

Begin

rs232_buffer[i]:=read_rs232(10); // read length x byte 10 ms timeout

if read_error then ......        // handle error

End;

Bytes in receiving Buffer for ISO9141-2  single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 06 | message length |
| [2] | 48 | Header1 |
| [3] | 6B | Header2 |
| [4] | 10 | Responding ECU address |
| [5] | 48 | Mode 08 response |
| [6] | 01 | TID |
| [7] | Chksum | Checksum of ISO message |

Bytes in receiving Buffer for J1850-VPW  single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 06 | message length |
| [2] | 48 | Header1 |
| [3] | 6B | Header2 |
| [4] | 10 | Responding ECU address |
| [5] | 48 | Mode 08 response |
| [6] | 01 | TID |
| [7] | CRC | CRC of j1850 message |

Bytes in receiving Buffer for J1850-PWM  single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 06 | message length |
| [2] | 41 | Header1 |
| [3] | 6B | Header2 |
| [4] | 10 | Responding ECU address |
| [5] | 48 | Mode 08 response |
| [6] | 01 | TID |
| [7] | CRC | CRC of j1850 message |

Bytes in receiving Buffer for KWP2000  single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 07 | message length |
| [2] | 82 | Header1 |
| [3] | F1 | Header2 |
| [4] | 10 | Responding ECU address |
| [5] | 48 | Mode 08 response |
| [6] | 01 | TID |
| [7] | Chksum | Checksum of KWP message |

Bytes in receiving Buffer for CAN – 11 single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 0A | message length |
| [2] | 07 | Header1 |
| [3] | E8 | Header2 |
| [4] | 02 | PCI |
| [5] | 48 | Mode 08 response |
| [6] | 01 | TID |
| [7] | 00 | Not used bytes must be filled with 00 |
| [8] | 00 | Not used bytes must be filled with 00 |
| [9] | 00 | Not used bytes must be filled with 00 |
| [10] | 00 | Not used bytes must be filled with 00 |
| [11] | 00 | Not used bytes must be filled with 00 |

Bytes in receiving Buffer for CAN – 29 single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 0C | message length |
| [2] | 18 | Header1 |
| [3] | DA | Header2 |
| [4] | F1 | Destination address |
| [5] | 10 | Source address |
| [6] | 02 | PCI |
| [7] | 48 | Mode 08 response |
| [8] | 01 | TID |
| [9] | 00 | Not used bytes must be filled with 00 |
| [10] | 00 | Not used bytes must be filled with 00 |
| [11] | 00 | Not used bytes must be filled with 00 |
| [12] | 00 | Not used bytes must be filled with 00 |
| [13] | 00 | Not used bytes must be filled with 00 |

MODE 9

The purpose of this mode is to enable the external test equipment to request vehicle specific vehicle information such as Vehicle Identification Number (VIN) and Calibration IDs. ECU indicates which INFOTYPEs are supported . INFOTYPE 00 is a bit-encoded value that indicates support for INFOTYPEs from 01 to 20. This is the same concept as used for PID support in Mode 01 .

Exemple how to get VIN from CAR

Handshaking in generic mode

write_rs232(02); 			    // Send generic access command 02

rs232_buffer[0]:=read_rs232(250);    // wait for ACK

if read_error then exit              // no response

if rs232_buffer[0] <> 06 then exit   // not ACK

Now send mode 09

write_rs232(09); 			    // Mode 09

write_rs232(02); 			    // Get VIN byte 1..4

Now get response  02 and response length

rs232_buffer[0]:=read_rs232(250);    // wait for 02

if read_error then exit              // no response

if rs232_buffer[0] <> 02 then exit   // false sync.

rs232_buffer[1]:=read_rs232(10);     // wait for 1.byte

if read_error then exit              // no response

if rs232_buffer[1] = 0

then ...........              // length = 00 handle error

Length := rs232_buffer[1];           // get incoming message length

For i:=1 to length do

Begin

rs232_buffer[i]:=read_rs232(10); // read length x byte 10 ms timeout

if read_error then ......        // handle error

End;

Bytes in receiving Buffer for ISO9141-2 , J1850 , KWP2000  single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 46 | message length |
| [2] | HH | Header 1 of protocol |
| [3] | HH | Header 2 of protocol |
| [4] | HH | Header 3 of protocol |
| [5] | 49 | Mode 09 response |
| [6] | 02 | Get VIN response |
| [7] | 01 | First message |
| [8] | 00 | -- |
| [9] | 00 | -- |
| [10] | 00 | -- |
| [11] | VV | VIN-byte 1 |
| [12] | Chksum | Checksum / CRC |

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [13] | HH | Header 1 of protocol |
| [14] | HH | Header 2 of protocol |
| [15] | HH | Header 3 of protocol |
| [16] | 49 | Mode 09 response |
| [17] | 02 | Get VIN response |
| [18] | 02 | 2. message |
| [19] | VV | VIN byte 2 |
| [20] | VV | VIN byte 3 |
| [21] | VV | VIN byte 4 |
| [22] | VV | VIN-byte 5 |
| [23] | Chksum | Checksum / CRC |

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [24] | HH | Header 1 of protocol |
| [25] | HH | Header 2 of protocol |
| [26] | HH | Header 3 of protocol |
| [27] | 49 | Mode 09 response |
| [28] | 02 | Get VIN response |
| [29] | 03 | 3. message |
| [30] | VV | VIN byte 6 |
| [31] | VV | VIN byte 7 |
| [32] | VV | VIN byte 8 |
| [33] | VV | VIN-byte 9 |
| [34] | Chksum | Checksum / CRC |

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [35] | HH | Header 1 of protocol |
| [36] | HH | Header 2 of protocol |
| [37] | HH | Header 3 of protocol |
| [38] | 49 | Mode 09 response |
| [39] | 02 | Get VIN response |
| [40] | 04 | 4. message |
| [41] | VV | VIN byte 10 |
| [42] | VV | VIN byte 11 |
| [43] | VV | VIN byte 12 |
| [44] | VV | VIN-byte 13 |
| [45] | Chksum | Checksum / CRC |

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [46] | HH | Header 1 of protocol |
| [47] | HH | Header 2 of protocol |
| [48] | HH | Header 3 of protocol |
| [49] | 49 | Mode 09 response |
| [50] | 02 | Get VIN response |
| [51] | 05 | 5. message |
| [52] | VV | VIN byte 14 |
| [53] | VV | VIN byte 15 |
| [54] | VV | VIN byte 16 |
| [55] | VV | VIN-byte 17 |
| [56] | Chksum | Checksum / CRC |

Bytes in receiving Buffer for CAN BUS  single ECU is responding

| RS232_buffer | Value | Remarque |
| --- | --- | --- |
| [0] | 02 | Response to generic command |
| [1] | 17 | message length |
| [2] | 07 | Header 1 of protocol |
| [3] | E8 | Header 2 of protocol |
| [4] | PP | PCI |
| [5] | 49 | Mode 09 response |
| [6] | 02 | Get VIN response |
| [7] | 01 | First message |
| [8] | VV | VIN-byte 1 |
| [9] | VV | VIN-byte 2 |
| [10] | VV | VIN-byte 3 |
| [11] | VV | VIN-byte 4 |
| [12] | VV | VIN-byte 5 |
| [13] | VV | VIN-byte 6 |
| [14] | VV | VIN-byte 7 |
| [15] | VV | VIN-byte 8 |
| [16] | VV | VIN-byte 9 |
| [17] | VV | VIN-byte 10 |
| [18] | VV | VIN-byte 11 |
| [19] | VV | VIN-byte 12 |
| [20] | VV | VIN-byte 13 |
| [21] | VV | VIN-byte 14 |
| [22] | VV | VIN-byte 15 |
| [23] | VV | VIN-byte 16 |
| [24] | VV | VIN-byte 17 |